Manish Singh Ranaa and Chandan Mahantab

aCorresponding authors at: Ph.D. research scholar, Department of Civil Engineering, Indian Institute of Technology, Guwahati, 781039; India; E-mail addresses: manishsingh@iitg.ac.in, mrana2744@gmail.com

bProfessor, Department of Civil Engineering, Indian Institute of Technology, Guwahati, 781039; India; E-mail addresses: chandan@iitg.ac.in.

Historical data of flash flood and trend analysis information for Uttarakhand, India

# Abstract

Historical data is always useful in interpreting any hazard-affected location. In this article historical data were gathered from various literature reviews, journals, newspapers, reports, and other sources to generate a flash flood map for Uttarakhand state, India. Between 1970 and 2020, a total of 122 sites were identified as being at risk of flash flooding. Moreover, several studies on rainfall trends at various scales have concluded that global warming is increasing extreme precipitation as well as extreme weather-related occurrences and risks. Therefore, high spatial resolution (0.25*0.25 degree) daily gridded rainfall data from the India Meteorological Department (IMD) was utilized to analyse the change in percentage from 1970 to 2020 for annual, pre-monsoon, monsoon, post-monsoon, and winter seasons.

# 1 Introduction

Analyzing past flash flood records is necessary to forecast future flood events in a given location (Manandhar 2010). As a result, the most crucial aspect in predicting future disaster occurrence is an inventory map, which can represent single or numerous incidents in a certain area. Most of the mapping has been done with sufficient flash flood records, however, the selected area is ungauged. Uttarakhand’s mountain ranges have some of the world's most inaccessible and remote peaks and valleys. 122 historical records have been gathered from various sources.

Climate change has resulted in changes in rainfall patterns as well as a rise in the frequency and magnitude of flash floods. To examine the influence of climate change, several studies on rainfall trends on various scales have been done. Numerous studies on a regional (Narayanan et al. 2013, Kundu et al. 2015) and national (Guhathakurta et al. 2011, Mondal et al. 2015) scale have been conducted in India utilizing long-term precipitation databases. However, when it comes to finding hidden trends and selecting acceptable approaches, there has been a gap in the analysis of extreme rainfall occurrences.

Since severe rainfall and cloudburst are the most common causes of flash floods in Uttarakhand, this study used machine learning models to map flash flood-prone areas. Subsequently, the trend of seasonal (pre-monsoon, monsoon, post-monsoon, and winter) and annual rainfall data was assessed to provide a long-term solution through various adaptation and mitigation strategies that may be used to deal with flash flood difficulties in the research area.

# 2 Materials and methods

## 2.1 Historical data of flash flood

Uttarakhand is selected as the study area which is located in the northern state of India. The study area is 53,483 km2 and is located between 28o 44' & 31o 28' N latitude and 77o 35' & 81o 01' E longitude. The historical data for flash flood mapping was mostly gathered from various literature reviews, publications, newspapers, reports, etc. Each flash flood's location is listed in Dataset 1. This data also includes loss and damage information from flash floods, as well as sources of flash flood data. To predict flash flood vulnerable areas, a new hybrid flash flood model employing Naive Bayes (NB), the quantitative approach of the weight of evidence (WOE), and adaptive boosting (AdaBoost) in R open-source software constructed using historical data and other conditioning factors.

Dataset 1. Historical data of flash flood of Uttarakhand.

| No. | Date | Location | Loss and damage (human, cattle, property, etc.) | Reference |
| --- | --- | --- | --- | --- |
| 1 | 20 July 1970 | Birahi Ganga, Chamoli | - | Rana et al. 2013 |
| 2 | 20 July 1970 | Ganai, Almora | Killing 200 persons while uncounted animals were lost and damaged 33 houses completely | Joshi and Kumar 2006 |
| 3 | 19 July 1971 | Dobata, Pithoragarh | Killed 12 people and buried 35 houses | Sajwan and Khanduri 2018 |
| 4 | 05 August 1978 | Kanoldiya Gad, Uttarkashi | Several villages were marooned, Maneri-Bhali HEP suffered heavily damaged and most of the shops and houses in Joshiara near Uttarkashi were swept away | Khanduri 2020 |
| 5 | 17 June 1979 | Saikot, Chamoli | Killing 3 persons while 70 animals were lost and damaged to 50 houses | Joshi and Kumar 2006 |
| 6 | 17 August 1979 | Kuntha, Ukhimath, Rudraprayag | Killing 39 persons while 39 animals were lost and damaged to 20 houses | Joshi and Kumar 2006 |
| 7 | 17 August 1979 | Sirwari, Rudraprayag | Killing 13 persons while 150 animals were lost and damaged to 34 houses | Joshi and Kumar 2006 |
| 8 | 25 June 1980 | Gyansu Nala, Uttarkashi | 24 persons were killed and washed away many houses in its vicinity | Khanduri 2020 |
| 9 | 31 July 1982 | Mandakhal Chennil, Pauri Garhwal | Killing 3 persons while 80 animals were lost and damaged to 8 houses | Joshi and Kumar 2006 |
| 10 | 22 July 1983 | Karmi-Kapkot, Bageshwar | 6 houses were washed away, 25 persons were missing and 20 cattle lost | Anbalagan 1996 |
| 11 | 1 August 1989 | Thalisain, Pauri | - | Khanduri et al. 2018 |
| 12 | 5 September 1989 | Karanprayag, Chamoli | 3 people died and two injured | Pandey and Vishwakarma 2020 |
| 13 | 01 August 1991 | Mandal, Chamoli | - | Khanduri et al. 2018 |
| 14 | 16 August 1991 | Dewar Khandora, Chamoli | 2 human casualties, 51 cattle lost and 4 houses perished | Joshi and Maikhuri 1997 |
| 15 | 26 December 1991 | Uttarkashi | 3 people died | Pandey and Vishwakarma 2020 |
| 16 | 02 September 1992 | Khankrakhet, Gadni, Chamoli | Killing 14 persons while damaging 31 houses | Joshi and Kumar 2006 |
| 17 | 10 July 1994 | Chaukhutia, Almora | 4 people died | Pandey and Vishwakarma 2020 |
| 18 | 13 August 1995 | Bhitai Malli, Pauri Garhwal | Killing 13 persons while damaging 6 houses | Joshi and Kumar 2006 |
| 19 | 17 July 1996 | Berinaga, Pithoragarh | Killing 18 persons while 85 animals lost | Joshi and Kumar 2006 |
| 20 | 26 July 1996 | Raintoli, Pithoragarh | Killed 16 villagers | Sajwan and Khanduri 2018 |
| 21 | 02 August 1997 | Near Neelkanth Mahadev Temple, Haridwar | 8 People died | Pandey and Mishra 2015 |
| 22 | 17 August 1998 | Malpa, Alaknanda | 221 people died | Sajwan and Khanduri 2018 |
| 23 | 19 August 1998 | Okhimath, Rudraprayag | Killing 103 persons while 422 animals were lost and damaged to 820 houses | Joshi and Kumar 2006 |
| 24 | 16 July 2001 | Phat-Byung, Ukhimath, Rudraprayag | Killing 27 persons while 64 animals were lost and damaged to 22 houses | Khanduri 2020 |
| 25 | 17 July 2001 | Near kunda, Rudraprayag | 27 people died | Pandey and Vishwakarma 2020 |
| 26 | 27 July 2001 | Khetgaon, Dharchula | Killed 5 people | Sajwan and Khanduri 2018 |
| 27 | 11 August 2001 | Phata, Rudraprayag | Killing 27 persons while 64 animals were lost and damaged to 22 houses | Joshi and Kumar 2006 |
| 28 | 31 August 2001 | Gona, Tehri | 7 persons were killed while 12 were injured together with 7 cattle lost and 28 houses completely damaged | Khanduri 2020 |
| 29 | 12 July 2002 | Khetgaon, Pithoragarh | Killing 4 people while 1 person missing and 20 animals were lost together with damage to 7 houses | Khanduri 2020 |
| 30 | 10 August 2002 | Boodha Kedar, Tehri | 28 persons were killed while 99 cattle were lost, 151 houses together with a micro-hydropower plant were damaged | Khanduri 2020 |
| 31 | 19 August 2002 | Bhatwari, Uttarkashi | Killing 5 people while 26 animals lost | Khanduri 2020 |
| 32 | 15 July 2003 | Didihat, Pithoragarh | 4 people died | Pandey and Vishwakarma 2020 |
| 33 | 29 August 2003 | Sarnol, Uttarkashi | 207 animals were lost | Khanduri 2020 |
| 34 | 21 May 2004 | Kapkot, Bageshwar | 6 people died | Pandey and Vishwakarma 2020 |
| 35 | 09 June 2004 | Kapkot, Bageshwar | - | Pandey and Vishwakarma 2020 |
| 36 | 06 July 2004 | Alaknanda river basin | At least 17 people were killed and 28 injured when three vehicles were swept into the Alaknanda river | Pandey and Vishwakarma 2020 |
| 37 | 14 September 2004 | Ranikhet | 1 people died | Pandey and Vishwakarma 2020 |
| 38 | 23 September 2004 | Amparav, Kathgodam, Nainital | Killing 3 persons while 7 residential and commercial structures were destroyed in the incident and a total of 8.9 hac agricultural fields were damaged | Khanduri 2020 |
| 39 | 24 September 2004 | Near Badrinath | A stretch of 200 Rishikesh-Badrinath road (NH 58) was completely damaged | Khanduri 2020 |
| 40 | 21 June 2005 | Teela, Thalisain, Garhwal | Killing 30 animals and 6 houses completely damaged | Khanduri 2020 |
| 41 | 30 June 2005 | Govindghat, Chamoli | 11 people lost their lives, and more than 200 vehicles, 20 shops, Hotels, and human settlements had been washed away | Khanduri 2020 |
| 42 | 21 July 2005 | Vijaynagar, Rudraprayag | 4 human lives were lost and 14 houses were destroyed | Khanduri 2020 |
| 43 | 21 July 2005 | Jhuni, Kapkot, Bageshwar | Killed 1 person while 3 cattle were lost and 6 houses destroyed | Khanduri 2020 |
| 44 | 17 August 2005 | Ramolsari, Tehri | - | DMP |
| 45 | 18 August 2005 | Semla tok of rathi village, dharchula, pithoragarh | 8 persons were killed while 129 animals lost | Khanduri 2020 |
| 46 | 27 August 2005 | Atholi village, Bhatwari, Uttarkashi | 15 animals were lost and 3 cow sheds destroyed | Khanduri 2020 |
| 47 | 26 July 2006 | Ladoli, Devali, Gholtir and Gurayal, Rudraprayag | 19 animals were killed while 6 shops and 8 houses were partially damaged | Khanduri 2020 |
| 48 | 11 August 2006 | Chamoli Tehsil, Chamoli | 18 mud houses were partially damaged | Khanduri 2020 |
| 49 | 12 July 2007 | Devpuri area, Chamoli | 5 persons were killed | Sajwan and Khanduri 2018 |
| 50 | 13 August 2007 | Didihat, Pithoragarh | 4 people died | Pandey and Vishwakarma 2020 |
| 51 | 01 September 2007 | Baram, Pithoragarh | 5 persons were killed and 9 others were feared dead | Khanduri 2020 |
| 52 | 06 September 2007 | Dharchula, Pithoragarh | 15 fatalities and loss of livestock | Pandey and Vishwakarma 2020 |
| 53 | 12 September 2007 | Baran, Pithoragarh | 9 persons killed | Sajwan and Khanduri 2018 |
| 54 | 25 July 2009 | Joshimath, Chamoli | 3 people died | NIDM |
| 55 | 08 August 2009 | Jhakhla and Lah, Pithoragarh | Wiped out 2 villages | Sajwan and Khanduri 2018 |
| 56 | 18 August 2009 | Munsyari, Pithoragarh | 43 people died | Pandey and Vishwakarma 2020 |
| 57 | 19 July 2010 | Kot, Pauri | 6 people died | NIDM |
| 58 | 18 August 2010 | Dhari, Nainital | 30 people died and 3 injured | NIDM |
| 59 | 18 August 2010 | Kapkot, Bageshwar | 18 school children were buried alive and 8 injured due to a massive cloudburst | Pandey and Vishwakarma 2020 |
| 60 | 08 September 2010 | Karanprayag, Chamoli | 3 people died | NIDM |
| 61 | 11 September 2010 | Nyalgarh, Pauri | 3 people died | NIDM |
| 62 | 18 September 2010 | sumgarh, Bageshwar | 18 children of classes I and II of a private primary school, Saraswati Shishu Mandir were buried alive under rock debris | Khanduri 2020 |
| 63 | 18 September 2010 | Jwalapur kasims, Haridwar | 5 people died and 2 injured | NIDM |
| 64 | 20 September 2010 | Dhari, Nainital | 5 people died and 12 injured | NIDM |
| 65 | 22 September 2010 | Kot, Pauri | 5 people died | NIDM |
| 66 | 06 May 2011 | Raipur, Dehradun | 3 people died | Pandey and Vishwakarma 2020 |
| 67 | 15 August 2011 | Tuneda, Bageshwar | 21 people died and 1 injured | Pandey and Vishwakarma 2020 |
| 68 | 03 August 2012 | Pandrasu ridge, Garhwal | 35 humans, 436 livestock lost and 591 houses damaged | Dimri et al. 2017 |
| 69 | 03 August 2012 | Asiganga, Uttarkashi | 36 human beings and 436 livestock dead, and affected about 12,000 people, damaging property worth about Rs 612 crore | Gupta et al. 2013 |
| 70 | 13 September 2012 | Ukhimath, Rudraprayag | 66 humans died | Dimri et al. 2017 |
| 71 | 16 June 2013 | Badrinath | The estimated death toll was more than 10,000 and still, 3000 persons were missing | Das 2013 |
| 72 | 16 June 2013 | Kedarnath | The estimated death toll was more than 10,000 and still, 3000 persons were missing | Das 2013 |
| 73 | 31 July 2014 | Tehri, Uttarakhand | At least 4 people were reported dead | Pandey and Vishwakarma 2020 |
| 74 | 28 May 2016 | Kemra, Kothiara, Tehri | 120 houses, 100 animals | Dimri et al. 2017 |
| 75 | 28 May 2016 | Siliara, Kothiara, Tehri | - | Dimri et al. 2017 |
| 76 | 28 May 2016 | Ghansali, Tehri | 6 people died, and at least 50 residential houses and more than 100 animals were buried | Sajwan et al. 2017 |
| 77 | 30 June 2016 | Ghat, Chamoli | 30 people were killed and 2 houses were washed away | Sajwan et al. 2017 |
| 78 | 01 July 2016 | Chamoli | 30 people were killed and Several homes in remote villages have been buried | Pandey and Mishra 2018 |
| 79 | 01 July 2016 | Bastari, Pithoragarh | At least 30 people were dead and several houses destroyed or washed away | Sajwan et al. 2017 |
| 80 | 26 May 2017 | Tatalgaon and Bijrani near Chukhutia | Life of loss of 8 domestic animals and one house buried | Khanduri 2020 |
| 81 | 14 August 2017 | Malpa, Pithoragarh | 8 Persons were killed while 10 people were missing and 4 shops were washed out | Sajwan and Khanduri 2018 |
| 82 | 02 July 2018 | Munsyari, Pithoragarh | Sheraghat Hydro Power Project damaged | SANDARP, July 21, 2018 |
| 83 | 11 July 2018 | Seema dwar, Dehradun | 7 people died, 2 were injured and 2 houses collapsed | SANDARP, July 21, 2018 |
| 84 | 16 July 2018 | Tharali, Chamoli | 2 ropeways, 1 road bridge washed away, 2 cattle died and several mini-hydro projects affected | SANDARP, July 21, 2018 |
| 85 | 17 July 2018 | Yamunotri, Uttarkashi | The footbridge connecting to the shrine was washed away and Kali Kamli Dharamshala was also severely damaged together with the hot water bath Kunds were filled with debris | Khanduri 2020 |
| 86 | 19 July 2018 | Joshimath, Chamoli | As many as 2 persons are dead and 5 persons are trapped in the debris and around 150 meters long Joshimath-Malari road stretch is washed away | Khanduri 2020 |
| 87 | 02 June 2019 | Lam Bagar, Chamoli | 1 person died and agricultural land damaged | SANDARP, December 11, 2019 |
| 88 | 02 June 2019 | Khira, Chaukhatiya, Almora | - | SANDARP, December 11, 2019 |
| 89 | 21 June 2019 | Market area, Mori, Uttarkashi | 1 person was dead and four were injured | SANDARP, December 11, 2019 |
| 90 | 23 June 2019 | Mansari, Thailisain, Pauri | - | SANDARP, December 11, 2019 |
| 91 | 04 July 2019 | Sari Chamshill, Augustmuni, Rudraprayag | About 60 m motor road was washed away, the water supply line was damaged and agricultural fields were washed away | Sati 2020 |
| 92 | 07 July 2019 | Paligarh tributary in Yamunotri, Uttarkashi | Machines were drowned, a big portion of the road was washed away and forestland damaged | Sati 2020 |
| 93 | 06 August 2019 | Pathar Katta, Gairsain, Chamoli | Damaged 4 cowsheds, 5 bridges, and a school building | SANDARP, December 11, 2019 |
| 94 | 06 August 2019 | Mayali, Rudraprayag | - | SANDARP, December 11, 2019 |
| 95 | 08 August 2019 | Padmalla, Chamoli | About 12 houses, two dozen cow sheds, 6-foot bridges, 10 water mills, water supply lines, and agricultural lands were washed away | Khanduri 2020 |
| 96 | 08 August 2019 | Saikot, Dasholi, Chamoli | killed about 4 people, and 42 cattle and damaged 30 houses, 24 cowsheds | SANDARP, December 11, 2019 |
| 97 | 08 August 2019 | Tharti, Bhilangna, Tehri | Many houses were damaged, a transformer and 8 poles were drowned | Sati 2020 |
| 98 | 08 August 2019 | Kuledi village, Kriti Nagar, Tehri | A bridge was collapsed | Sati 2020 |
| 99 | 08 August 2019 | Faldia village, Pindar River Basin | A total of 12 houses and 8cowsheds were fully damaged, agricultural land and three bridges were washed away | Sati 2020 |
| 100 | 08 August 2019 | Maithana village, Alaknanda River Basin | 2 houses were collapsed | Sati 2020 |
| 101 | 08 August 2019 | Nandgaon, Pathargarh in Purola, Uttarkashi | Agricultural land was washed away and existing crops were damaged | Sati 2020 |
| 102 | 12 August 2019 | Ghat, Chamoli | - | SANDARP, December 11, 2019 |
| 103 | 12 August 2019 | Ali, Ghat, Chamoli | 4 people died | Sati 2020 |
| 104 | 12 August 2019 | Lankhi, Ghat, Chamoli | 2 people died | Sati 2020 |
| 105 | 12 August 2019 | Hawel Kulwan, Garud, Bageshwar | - | SANDARP, December 11, 2019 |
| 106 | 12 August 2019 | Banjbazar, Chamoli | 6 people had died and 40 animals were drowned | Sati 2020 |
| 107 | 14 August 2019 | Bangaan and Mori, Uttarkashi | Heavy losses of property and agricultural land took place. Among them, 16 shops were damaged. | Sati 2020 |
| 108 | 17 August 2019 | Sanel village, Uttarkashi | A crusher machine drowned and forest land degraded | Sati 2020 |
| 109 | 18 August 2019 | Makudi, Mori, Uttarkashi | 21 persons were killed and 74 animals lost, 2 motor bridges are damaged along with 2 pedestrian bridges | Khanduri 2020 |
| 110 | 18 August 2019 | Chauthan, Thailisain, Pouri | - | SANDARP, December 11, 2019 |
| 111 | 18 August 2019 | Makuri, Uttarkashi | 17 people died | SANDARP, December 11, 2019 |
| 112 | 18 August 2019 | Gaurikund, Rudraprayag district | 3 bridges were collapsed | Sati 2020 |
| 113 | 19 August 2019 | Ralakot and Chirabari villages, Pratap Nagar, Tehri | 4 houses were damaged and 2 shops were collapsed | Sati 2020 |
| 114 | 20 August 2019 | Arakot, Uttarkashi | Agricultural land was washed away and existing crops were damaged | Sati 2020 |
| 115 | 02 September 2019 | Jajar village, Dhauladevi, Almora | - | SANDARP, December 11, 2019 |
| 116 | 06 September 2019 | Kumbhichuad and Rampur (Jamun Srot), Kotdwar | Several houses were damaged and agricultural land was washed away | Sati 2020 |
| 117 | 07 September 2019 | Timtia, Munsyari, Pithoragarh | 1 person died | SANDARP, December 11, 2019 |
| 118 | 07 September 2019 | Govind Ghat, Chamoli | Several vehicles were buried under debris while about 30 meters stretch of Rishikesh-Badrinath road (NH 58) was completely damaged | Khanduri 2020 |
| 119 | 07 September 2019 | Gudam, Tharali, Chamoli | 4 houses were partially damaged, agricultural land was drowned and debris entered into more than 12 houses and cowsheds. | Sati 2020 |
| 120 | 07 September 2019 | Dhurma village, Ghat, Mandakini River Basin | Debris entered the middle of the village and damaged many houses and cowsheds and washed away a large proportion of agricultural land | Sati 2020 |
| 121 | 08 September 2019 | Dhurma, Ghat, Chamoli | A house was washed away, while 6 others were damaged, inter-college was damaged and two water mills were washed away | SANDARP, December 11, 2019 |
| 122 | 27 September 2019 | Clement, Dehradun | 15 houses were damaged | SANDARP, December 11, 2019 |

## 2.2 Rainfall pattern analysis

IMD rainfall gridded dataset is prepared at the National Data Centre, IMD, Pune, which uses daily rainfall data of 6329 stations. 537 of the 6329 stations are IMD observatory stations, however, 522 are part of the Hydro-meteorology program and 70 are under the Agriculture meteorological stations (Singh et al. 2019), and the remaining stations are run by state governments. Daily rainfall data obtained from IMD has a spatial resolution of 25 km. In the present study, 84 rainfall grids were used to assess the trend of seasonal (pre-monsoon, monsoon, post-monsoon, and winter) and annual rainfall series using Mann-Kendall, Modified Mann-Kendall with bias correction of coefficient, and Innovative Trend Analysis (ITA) in the flash-flood scenario.

The Changes were calculated as a percentage change over 84 grid points (Dataset 5, 6, and 7) and were interpolated using the inverse distance weighting (IDW) method to show the spatial pattern of rainfall trends using ArcGIS tool. Z statistics of MK and Modified MK were estimated to check the significance level for annual and seasonal rainfall series for each grid at 95% significance level (Dataset 2 and 3). Furthermore, the significance of the data was determined using the ITA method with a trend slope of 95% significance (Dataset 4).

Compared to the MK and Modified MK tests, the ITA approach provides more information regarding the trends of annual and seasonal rainfall data in evaluating multiple categories of values. Moreover, the ITA approach solves problems such as independent time series structure, the normalcy of distribution, and data length. The findings revealed that flash floods are more common in the central region of the study area due to an increasing tendency in rainfall patterns throughout the monsoon season.

Dataset 2. Mann-Kendall Z statistics for annual and seasonal changes of precipitation for Uttarakhand.

| 1970-2020 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Z-value (Mann-Kendall test) |  |  |  |  |  |  |  |
| Grid No. | Lat | Long | Annual | Pre-Monsoon | Monsoon | Post-monsoon | Winter |
| 1 | 31.25 | 79 | -0.699 | -1.413 | 0.195 | -1.836 | -2.55 |
| 2 | 31.25 | 79.25 | -1.235 | -1.803 | 0.162 | -1.917 | -3.103 |
| 3 | 31 | 78 | -2.567 | -1.633 | -2.355 | -1.812 | -1.933 |
| 4 | 31 | 78.25 | -1.072 | -0.828 | -0.877 | -1.04 | -0.861 |
| 5 | 31 | 78.5 | 0.138 | 0.032 | 0.114 | -1.478 | -0.617 |
| 6 | 31 | 78.75 | -0.008 | 0 | -0.292 | -1.023 | -0.154 |
| 7 | 31 | 79 | -0.276 | -1.332 | 0.479 | -1.316 | -2.307 |
| 8 | 31 | 79.25 | -0.536 | -1.771 | 0.975 | -2.234 | -2.875 |
| 9 | 31 | 79.5 | -0.357 | -1.754 | 1.292 | -2.429 | -2.583 |
| 10 | 31 | 79.75 | 0.276 | -1.365 | 2.745 | -2.575 | -2.697 |
| 11 | 30.75 | 77.75 | -2.372 | 0.041 | -1.852 | -1.47 | -0.081 |
| 12 | 30.75 | 78 | -1.771 | -0.333 | -1.121 | -1.275 | 0.569 |
| 13 | 30.75 | 78.25 | 1.706 | 0.666 | 1.365 | -0.097 | 0.95 |
| 14 | 30.75 | 78.5 | -0.244 | 0.617 | -0.244 | -1.365 | 0.463 |
| 15 | 30.75 | 78.75 | 0.455 | 0.211 | 0.26 | -0.43 | 0.382 |
| 16 | 30.75 | 79 | -0.682 | -1.17 | 0.292 | -1.186 | -1.511 |
| 17 | 30.75 | 79.25 | -0.715 | -1.966 | 1.04 | -2.356 | -2.892 |
| 18 | 30.75 | 79.5 | -1.105 | -1.917 | 1.007 | -2.916 | -2.924 |
| 19 | 30.75 | 79.75 | 1.267 | -1.097 | 3.834 | -2.607 | -2.713 |
| 20 | 30.75 | 80 | 1.917 | 0.114 | 3.298 | -2.477 | -2.461 |
| 21 | 30.5 | 77.75 | 1.787 | 1.34 | 1.73 | -0.796 | 1.056 |
| 22 | 30.5 | 78 | -0.341 | -0.349 | -0.39 | -0.958 | 0.081 |
| 23 | 30.5 | 78.25 | -0.292 | -0.179 | -0.227 | -1.592 | 0.333 |
| 24 | 30.5 | 78.5 | -0.081 | -0.114 | 0.227 | -2.128 | 0.926 |
| 25 | 30.5 | 78.75 | 1.714 | 0.699 | 1.486 | 0.097 | 0.625 |
| 26 | 30.5 | 79 | 0.146 | -0.463 | 0.617 | -0.187 | -0.828 |
| 27 | 30.5 | 79.25 | 0.942 | -1.283 | 2.014 | -1.998 | -1.998 |
| 28 | 30.5 | 79.5 | -0.601 | -1.543 | 0.934 | -2.583 | -2.697 |
| 29 | 30.5 | 79.75 | 2.94 | 0.179 | 4.435 | -2.355 | -2.047 |
| 30 | 30.5 | 80 | 2.193 | 0.893 | 2.729 | -2.112 | -1.292 |
| 31 | 30.5 | 80.25 | 1.072 | 0.942 | 1.495 | -1.949 | -0.828 |
| 32 | 30.25 | 78 | 1.251 | 1.706 | 1.007 | -0.041 | 0.967 |
| 33 | 30.25 | 78.25 | 1.023 | 2.226 | 0.057 | -0.609 | 0.942 |
| 34 | 30.25 | 78.5 | 2.006 | 1.405 | 1.949 | -1.048 | 1.714 |
| 35 | 30.25 | 78.75 | 2.469 | 1.446 | 2.68 | -0.585 | 1.267 |
| 36 | 30.25 | 79 | 2.729 | 1.088 | 2.973 | 0.699 | 0.13 |
| 37 | 30.25 | 79.25 | 2.729 | 0.763 | 2.973 | -0.236 | -0.268 |
| 38 | 30.25 | 79.5 | 2.209 | -0.958 | 3.882 | -2.64 | -2.51 |
| 39 | 30.25 | 79.75 | 2.209 | 0.268 | 2.875 | -2.835 | -1.787 |
| 40 | 30.25 | 80 | 0.682 | 1.113 | 1.527 | -1.486 | -1.153 |
| 41 | 30.25 | 80.25 | 0.227 | 1.105 | 0.617 | -1.722 | -0.666 |
| 42 | 30.25 | 80.5 | 0.617 | 1.048 | 0.699 | -2.063 | -0.471 |
| 43 | 30.25 | 80.75 | 0.747 | 0.999 | 0.796 | -1.949 | -0.146 |
| 44 | 30.25 | 81 | 0.861 | 1.153 | 0.975 | -2.079 | 0.171 |
| 45 | 30 | 77.75 | 1.478 | 1.486 | 0.991 | -0.82 | 0.349 |
| 46 | 30 | 78 | 1.933 | 2.697 | 1.243 | 0.211 | 0.666 |
| 47 | 30 | 78.25 | 0.13 | 1.43 | -0.43 | -0.512 | 0.699 |
| 48 | 30 | 78.5 | -0.341 | 0.942 | -0.975 | -1.154 | 1.511 |
| 49 | 30 | 78.75 | 0.796 | 1.535 | 0.276 | -0.853 | 1.381 |
| 50 | 30 | 79 | 0.56 | 0.65 | 0.91 | -1.397 | 0.049 |
| 51 | 30 | 79.25 | 1.023 | 0.309 | 1.624 | -1.316 | -0.374 |
| 52 | 30 | 79.5 | 3.233 | 1.348 | 4.159 | -1.714 | 0.325 |
| 53 | 30 | 79.75 | 1.495 | 0.552 | 2.226 | -1.657 | -0.366 |
| 54 | 30 | 80 | 0.536 | 1.145 | 1.462 | -1.3 | -0.292 |
| 55 | 30 | 80.25 | 0.569 | 1.145 | 0.715 | -2.282 | 0.114 |
| 56 | 30 | 80.5 | 0.487 | 1.235 | 0.569 | -2.421 | 0.244 |
| 57 | 29.75 | 78 | 0.162 | 1.811 | 0 | -1.097 | 0.479 |
| 58 | 29.75 | 78.25 | 1.121 | 1.17 | 1.332 | -1.015 | 0.828 |
| 59 | 29.75 | 78.5 | -2.128 | 0.317 | -2.079 | -1.268 | 0.528 |
| 60 | 29.75 | 78.75 | -2.534 | -0.439 | -2.778 | -1.966 | 0.106 |
| 61 | 29.75 | 79 | -2.193 | -0.057 | -2.615 | -2.616 | -0.39 |
| 62 | 29.75 | 79.25 | -1.56 | -0.13 | -1.568 | -2.242 | -0.374 |
| 63 | 29.75 | 79.5 | 1.462 | 2.258 | 1.43 | -0.869 | 1.543 |
| 64 | 29.75 | 79.75 | 0.812 | 1.243 | 0.536 | -1.04 | 1.227 |
| 65 | 29.75 | 80 | 0.341 | 1.015 | -0.097 | -0.634 | 1.056 |
| 66 | 29.75 | 80.25 | 0.146 | 0.682 | -0.081 | -2.323 | 0.731 |
| 67 | 29.5 | 78.75 | -3.135 | -1.113 | -3.33 | -2.429 | -0.999 |
| 68 | 29.5 | 79 | -2.291 | -0.512 | -2.404 | -2.145 | -0.504 |
| 69 | 29.5 | 79.25 | -2.209 | -0.585 | -2.274 | -2.355 | -0.471 |
| 70 | 29.5 | 79.5 | -0.292 | 0.032 | -0.422 | -2.161 | 0.39 |
| 71 | 29.5 | 79.75 | 0.552 | 1.023 | 0.422 | -1.665 | 0.991 |
| 72 | 29.5 | 80 | 0.942 | 2.339 | 0.487 | -1.145 | 1.917 |
| 73 | 29.5 | 80.25 | 0.715 | 1.649 | 0.081 | -1.162 | 1.706 |
| 74 | 29.25 | 79 | -2.404 | -0.422 | -2.404 | -1.885 | -0.357 |
| 75 | 29.25 | 79.25 | 1.218 | 1.357 | 1.105 | -1.186 | 0.78 |
| 76 | 29.25 | 79.5 | 1.852 | 1.389 | 1.478 | -1.519 | 0.975 |
| 77 | 29.25 | 79.75 | -0.861 | 0.763 | -0.885 | -1.43 | 0.861 |
| 78 | 29.25 | 80 | 0.032 | 1.08 | 0.049 | -0.918 | 1.023 |
| 79 | 29.25 | 80.25 | 0.422 | 1.381 | 0.081 | -1.348 | 1.332 |
| 80 | 29 | 79.25 | 2.908 | 1.454 | 3.086 | -0.471 | 1.162 |
| 81 | 29 | 79.5 | 4.386 | 3.249 | 4.289 | 0.975 | 2.21 |
| 82 | 29 | 79.75 | 0.763 | 2.226 | 0.504 | -0.325 | 1.641 |
| 83 | 29 | 80 | 1.641 | 1.657 | 1.624 | -0.333 | 1.04 |
| 84 | 28.75 | 80 | 1.153 | 0.414 | 1.397 | -1.243 | 0.763 |

Dataset 3. Modified Mann-Kendall Z statistics for annual and seasonal changes of precipitation for Uttarakhand.

| 1970-2020 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Z-value (Modified Mann-Kendall) |  |  |  |  |  |  |  |
| Grid No. | Lat | Long | Annual | Pre-Monsoon | Monsoon | Post-monsoon | Winter |
| 1 | 31.25 | 79 | -0.903 | -1.405 | 0.251 | -1.941 | -2.443 |
| 2 | 31.25 | 79.25 | -1.004 | -1.455 | 0.201 | -1.89 | -2.359 |
| 3 | 31 | 78 | -2.275 | -1.64 | -2.309 | -1.757 | -1.74 |
| 4 | 31 | 78.25 | -1.037 | -1.071 | -0.736 | -0.87 | -0.937 |
| 5 | 31 | 78.5 | 0.184 | -0.017 | 0.552 | -1.355 | -0.753 |
| 6 | 31 | 78.75 | -0.217 | -0.284 | -0.05 | -0.987 | -0.1 |
| 7 | 31 | 79 | -0.519 | -1.355 | 0.452 | -1.706 | -2.125 |
| 8 | 31 | 79.25 | -0.736 | -1.472 | 0.636 | -2.175 | -2.091 |
| 9 | 31 | 79.5 | -0.719 | -1.656 | 0.92 | -2.359 | -2.041 |
| 10 | 31 | 79.75 | -0.167 | -1.322 | 2.074 | -2.309 | -2.024 |
| 11 | 30.75 | 77.75 | -2.61 | 0.217 | -1.656 | -1.004 | 0.067 |
| 12 | 30.75 | 78 | -2.091 | -0.569 | -1.087 | -0.82 | 0.502 |
| 13 | 30.75 | 78.25 | 1.623 | 0.703 | 1.623 | -0.268 | 1.004 |
| 14 | 30.75 | 78.5 | -0.05 | 0.569 | -0.017 | -1.171 | 0.535 |
| 15 | 30.75 | 78.75 | 0.151 | -0.017 | 0.234 | -0.803 | 0.485 |
| 16 | 30.75 | 79 | -0.335 | -1.171 | 0.201 | -1.338 | -1.489 |
| 17 | 30.75 | 79.25 | -0.552 | -1.74 | 0.619 | -2.192 | -2.192 |
| 18 | 30.75 | 79.5 | -1.004 | -1.84 | 0.736 | -2.275 | -2.125 |
| 19 | 30.75 | 79.75 | 0.753 | -1.037 | 3.312 | -2.225 | -1.991 |
| 20 | 30.75 | 80 | 1.188 | -0.1 | 2.208 | -1.957 | -1.322 |
| 21 | 30.5 | 77.75 | 1.957 | 0.887 | 1.539 | 0.1 | 1.071 |
| 22 | 30.5 | 78 | -0.268 | -0.134 | -0.167 | -0.753 | 0.167 |
| 23 | 30.5 | 78.25 | -0.084 | -0.217 | -0.084 | -1.422 | 0.418 |
| 24 | 30.5 | 78.5 | -0.151 | -0.05 | 0.067 | -1.824 | 0.084 |
| 25 | 30.5 | 78.75 | 1.573 | 0.351 | 1.288 | -0.268 | 0.87 |
| 26 | 30.5 | 79 | 0.201 | -0.836 | 0.652 | -0.468 | -0.669 |
| 27 | 30.5 | 79.25 | 0.636 | -1.472 | 1.89 | -2.041 | -1.706 |
| 28 | 30.5 | 79.5 | -0.954 | -1.405 | 0.669 | -2.476 | -2.125 |
| 29 | 30.5 | 79.75 | 2.242 | -0.301 | 3.731 | -2.125 | -1.723 |
| 30 | 30.5 | 80 | 0.753 | 0.385 | 1.439 | -1.84 | -0.786 |
| 31 | 30.5 | 80.25 | 0.602 | 1.121 | 0.82 | -1.673 | -0.084 |
| 32 | 30.25 | 78 | 1.221 | 1.589 | 1.305 | 0.251 | 0.853 |
| 33 | 30.25 | 78.25 | 1.121 | 1.623 | 0.452 | -0.586 | 1.054 |
| 34 | 30.25 | 78.5 | 1.807 | 1.205 | 1.506 | -1.037 | 1.874 |
| 35 | 30.25 | 78.75 | 2.576 | 1.305 | 2.66 | -0.619 | 1.439 |
| 36 | 30.25 | 79 | 2.744 | 0.97 | 2.76 | 0.552 | 0.402 |
| 37 | 30.25 | 79.25 | 2.66 | 0.569 | 2.76 | -0.502 | 0.05 |
| 38 | 30.25 | 79.5 | 1.673 | -0.87 | 3.279 | -2.543 | -1.773 |
| 39 | 30.25 | 79.75 | 1.121 | 0.77 | 1.807 | -1.874 | -0.284 |
| 40 | 30.25 | 80 | 0.736 | 0.87 | 0.887 | -1.221 | -0.067 |
| 41 | 30.25 | 80.25 | 0.703 | 0.937 | 0.435 | -1.389 | -0.368 |
| 42 | 30.25 | 80.5 | 0.619 | 0.903 | 0.452 | -1.773 | 0.234 |
| 43 | 30.25 | 80.75 | 0.569 | 0.887 | 0.385 | -1.673 | 0.402 |
| 44 | 30.25 | 81 | 0.519 | 0.719 | 0.351 | -2.008 | 0.268 |
| 45 | 30 | 77.75 | 1.322 | 1.422 | 0.887 | -0.619 | 0.468 |
| 46 | 30 | 78 | 1.924 | 2.242 | 1.288 | 0.201 | 0.87 |
| 47 | 30 | 78.25 | 0.619 | 1.071 | 0.017 | -0.586 | 1.221 |
| 48 | 30 | 78.5 | -0.268 | 0.736 | -0.87 | -1.121 | 1.907 |
| 49 | 30 | 78.75 | 0.853 | 1.506 | 0.318 | -0.77 | 1.74 |
| 50 | 30 | 79 | 0.669 | 0.686 | 1.087 | -1.472 | 0.351 |
| 51 | 30 | 79.25 | 1.071 | 0.217 | 1.723 | -1.489 | 0.067 |
| 52 | 30 | 79.5 | 2.995 | 1.104 | 3.413 | -1.506 | 0.318 |
| 53 | 30 | 79.75 | 1.623 | 1.054 | 1.506 | -1.706 | 0.201 |
| 54 | 30 | 80 | 0.719 | 0.92 | 0.669 | -0.937 | 0.284 |
| 55 | 30 | 80.25 | 0.485 | 0.77 | 0.418 | -2.008 | 0.335 |
| 56 | 30 | 80.5 | 0.719 | 0.786 | 0.502 | -2.225 | 0.318 |
| 57 | 29.75 | 78 | 0.535 | 1.69 | -0.084 | -0.903 | 0.552 |
| 58 | 29.75 | 78.25 | 1.188 | 0.836 | 1.338 | -1.004 | 0.97 |
| 59 | 29.75 | 78.5 | -1.723 | 0.485 | -1.673 | -1.104 | 0.736 |
| 60 | 29.75 | 78.75 | -1.941 | -0.318 | -2.342 | -1.941 | 0.151 |
| 61 | 29.75 | 79 | -1.857 | 0.184 | -2.225 | -2.627 | 0.067 |
| 62 | 29.75 | 79.25 | -1.455 | 0.167 | -1.221 | -2.392 | 0.033 |
| 63 | 29.75 | 79.5 | 1.455 | 1.121 | 1.405 | -0.753 | 1.338 |
| 64 | 29.75 | 79.75 | 0.669 | 0.853 | 0.468 | -0.878 | 0.937 |
| 65 | 29.75 | 80 | 0.786 | 0.619 | 0.519 | -0.033 | 1.188 |
| 66 | 29.75 | 80.25 | 0.502 | 0.619 | 0.301 | -2.091 | 0.703 |
| 67 | 29.5 | 78.75 | -3.062 | -0.636 | -3.346 | -2.409 | -0.284 |
| 68 | 29.5 | 79 | -1.74 | -0.134 | -1.907 | -2.342 | -0.151 |
| 69 | 29.5 | 79.25 | -1.589 | -0.284 | -1.606 | -2.576 | -0.151 |
| 70 | 29.5 | 79.5 | -0.418 | 0.151 | -0.602 | -2.158 | 0.318 |
| 71 | 29.5 | 79.75 | 0.502 | 0.502 | 0.017 | -1.271 | 0.82 |
| 72 | 29.5 | 80 | -0.184 | 1.238 | -0.084 | -1.004 | 1.338 |
| 73 | 29.5 | 80.25 | 0.301 | 0.703 | 0.184 | -0.803 | 1.355 |
| 74 | 29.25 | 79 | -2.426 | 0.033 | -2.259 | -2.008 | -0.151 |
| 75 | 29.25 | 79.25 | 1.372 | 1.188 | 1.154 | -1.121 | 0.77 |
| 76 | 29.25 | 79.5 | 1.991 | 1.004 | 1.623 | -1.355 | 0.954 |
| 77 | 29.25 | 79.75 | -0.669 | 0.385 | -0.92 | -1.322 | 1.121 |
| 78 | 29.25 | 80 | -0.385 | 0.786 | -0.485 | -0.836 | 1.054 |
| 79 | 29.25 | 80.25 | -0.234 | 0.736 | -0.067 | -0.954 | 1.271 |
| 80 | 29 | 79.25 | 3.262 | 1.188 | 3.614 | -0.251 | 0.686 |
| 81 | 29 | 79.5 | 3.848 | 2.71 | 4.049 | 1.188 | 1.464 |
| 82 | 29 | 79.75 | 0.502 | 1.924 | 0.251 | -0.435 | 1.723 |
| 83 | 29 | 80 | 1.255 | 1.171 | 1.154 | -0.519 | 1.238 |
| 84 | 28.75 | 80 | 0.853 | 0.151 | 1.238 | -1.255 | 1.037 |

Dataset 4. Innovative Trend Analysis trend slope statistics for annual and seasonal changes of precipitation for Uttarakhand.

| Grid No. | Lat | Long | 1970-2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Annual (ITA) |  |  | Post-monsoon (ITA) |  |  |
|  |  |  | s | CLlower95 | CLupper95 | s | CLlower95 | CLupper95 |
| 1 | 31.25 | 79 | -4.356 | -0.93 | 0.93 | -1.165 | -0.124 | 0.124 |
| 2 | 31.25 | 79.25 | -5.913 | -0.934 | 0.934 | -1.358 | -0.118 | 0.118 |
| 3 | 31 | 78 | -14.662 | -1.435 | 1.435 | -1.614 | -0.178 | 0.178 |
| 4 | 31 | 78.25 | -6.063 | -1.182 | 1.182 | -1.394 | -0.077 | 0.077 |
| 5 | 31 | 78.5 | 0.592 | -0.815 | 0.815 | -0.954 | -0.181 | 0.181 |
| 6 | 31 | 78.75 | -2.095 | -1.706 | 1.706 | -0.825 | -0.214 | 0.214 |
| 7 | 31 | 79 | -3.244 | -0.89 | 0.89 | -1.018 | -0.127 | 0.127 |
| 8 | 31 | 79.25 | -5.124 | -0.619 | 0.619 | -1.496 | -0.175 | 0.175 |
| 9 | 31 | 79.5 | -4.024 | -0.772 | 0.772 | -1.657 | -0.199 | 0.199 |
| 10 | 31 | 79.75 | -0.468 | -0.513 | 0.513 | -1.567 | -0.195 | 0.195 |
| 11 | 30.75 | 77.75 | -9.406 | -0.57 | 0.57 | -1.723 | -0.116 | 0.116 |
| 12 | 30.75 | 78 | -6.479 | -1.174 | 1.174 | -1.304 | -0.112 | 0.112 |
| 13 | 30.75 | 78.25 | 6.138 | -0.895 | 0.895 | -0.785 | -0.144 | 0.144 |
| 14 | 30.75 | 78.5 | 6.312 | -1.649 | 1.649 | -1.012 | -0.11 | 0.11 |
| 15 | 30.75 | 78.75 | -0.205 | -1.189 | 1.189 | -0.392 | -0.159 | 0.159 |
| 16 | 30.75 | 79 | -6.363 | -0.74 | 0.74 | -0.924 | -0.093 | 0.093 |
| 17 | 30.75 | 79.25 | -5.712 | -0.785 | 0.785 | -1.563 | -0.194 | 0.194 |
| 18 | 30.75 | 79.5 | -5.967 | -0.679 | 0.679 | -1.981 | -0.239 | 0.239 |
| 19 | 30.75 | 79.75 | 2.887 | -0.543 | 0.543 | -1.487 | -0.196 | 0.196 |
| 20 | 30.75 | 80 | 15.216 | -1.134 | 1.134 | -1.067 | -0.187 | 0.187 |
| 21 | 30.5 | 77.75 | 6.9 | -1.057 | 1.057 | -0.531 | -0.2 | 0.2 |
| 22 | 30.5 | 78 | -3.962 | -1.745 | 1.745 | -0.898 | -0.102 | 0.102 |
| 23 | 30.5 | 78.25 | 2.942 | -1.243 | 1.243 | -1.084 | -0.15 | 0.15 |
| 24 | 30.5 | 78.5 | 3.027 | -0.736 | 0.736 | -1.134 | -0.112 | 0.112 |
| 25 | 30.5 | 78.75 | 1.471 | -0.736 | 0.736 | -0.246 | -0.143 | 0.143 |
| 26 | 30.5 | 79 | -8.203 | -0.99 | 0.99 | -0.779 | -0.137 | 0.137 |
| 27 | 30.5 | 79.25 | 1.541 | -0.516 | 0.516 | -1.094 | -0.084 | 0.084 |
| 28 | 30.5 | 79.5 | -3.922 | -0.575 | 0.575 | -1.471 | -0.138 | 0.138 |
| 29 | 30.5 | 79.75 | 11.654 | -0.723 | 0.723 | -1.007 | -0.119 | 0.119 |
| 30 | 30.5 | 80 | 21.237 | -1.214 | 1.214 | -0.809 | -0.163 | 0.163 |
| 31 | 30.5 | 80.25 | 17.012 | -2.446 | 2.446 | -0.784 | -0.18 | 0.18 |
| 32 | 30.25 | 78 | 1.586 | -0.966 | 0.966 | -0.465 | -0.07 | 0.07 |
| 33 | 30.25 | 78.25 | -1.774 | -0.791 | 0.791 | -0.611 | -0.14 | 0.14 |
| 34 | 30.25 | 78.5 | 8.029 | -1.011 | 1.011 | -0.277 | -0.103 | 0.103 |
| 35 | 30.25 | 78.75 | 9.779 | -0.91 | 0.91 | 0.176 | -0.164 | 0.164 |
| 36 | 30.25 | 79 | 5.836 | -0.983 | 0.983 | -0.401 | -0.089 | 0.089 |
| 37 | 30.25 | 79.25 | 4.498 | -0.517 | 0.517 | -0.712 | -0.14 | 0.14 |
| 38 | 30.25 | 79.5 | 4.209 | -0.624 | 0.624 | -1.322 | -0.206 | 0.206 |
| 39 | 30.25 | 79.75 | 18.8 | -1.419 | 1.419 | -0.839 | -0.137 | 0.137 |
| 40 | 30.25 | 80 | 24.258 | -2.564 | 2.564 | -0.504 | -0.078 | 0.078 |
| 41 | 30.25 | 80.25 | 16.684 | -2.862 | 2.862 | -0.701 | -0.158 | 0.158 |
| 42 | 30.25 | 80.5 | 12.224 | -3.127 | 3.127 | -0.893 | -0.143 | 0.143 |
| 43 | 30.25 | 80.75 | 11.709 | -3.127 | 3.127 | -0.873 | -0.16 | 0.16 |
| 44 | 30.25 | 81 | 11.31 | -2.925 | 2.925 | -0.789 | -0.124 | 0.124 |
| 45 | 30 | 77.75 | 5.445 | -0.6 | 0.6 | -0.46 | -0.093 | 0.093 |
| 46 | 30 | 78 | 4.772 | -0.853 | 0.853 | 0.003 | -0.178 | 0.178 |
| 47 | 30 | 78.25 | -5.484 | -0.786 | 0.786 | -0.417 | -0.157 | 0.157 |
| 48 | 30 | 78.5 | -0.987 | -0.895 | 0.895 | -0.708 | -0.091 | 0.091 |
| 49 | 30 | 78.75 | 2.975 | -0.63 | 0.63 | -0.42 | -0.124 | 0.124 |
| 50 | 30 | 79 | 1.232 | -0.527 | 0.527 | -0.57 | -0.111 | 0.111 |
| 51 | 30 | 79.25 | 1.927 | -0.592 | 0.592 | -0.881 | -0.086 | 0.086 |
| 52 | 30 | 79.5 | 14.067 | -0.782 | 0.782 | -0.646 | -0.106 | 0.106 |
| 53 | 30 | 79.75 | 19.172 | -1.75 | 1.75 | -0.574 | -0.189 | 0.189 |
| 54 | 30 | 80 | 24.539 | -2.447 | 2.447 | -0.323 | -0.136 | 0.136 |
| 55 | 30 | 80.25 | 10.245 | -3.036 | 3.036 | -0.975 | -0.102 | 0.102 |
| 56 | 30 | 80.5 | 8.406 | -2.973 | 2.973 | -1.052 | -0.083 | 0.083 |
| 57 | 29.75 | 78 | -2.139 | -0.504 | 0.504 | -0.464 | -0.1 | 0.1 |
| 58 | 29.75 | 78.25 | -1.444 | -0.913 | 0.913 | -0.677 | -0.13 | 0.13 |
| 59 | 29.75 | 78.5 | -6.425 | -1.262 | 1.262 | -1.243 | -0.088 | 0.088 |
| 60 | 29.75 | 78.75 | -11.865 | -1.155 | 1.155 | -1.22 | -0.211 | 0.211 |
| 61 | 29.75 | 79 | -8.626 | -0.796 | 0.796 | -1.308 | -0.329 | 0.329 |
| 62 | 29.75 | 79.25 | -5.072 | -0.746 | 0.746 | -1.188 | -0.242 | 0.242 |
| 63 | 29.75 | 79.5 | 8.249 | -1.333 | 1.333 | -0.437 | -0.12 | 0.12 |
| 64 | 29.75 | 79.75 | 9.187 | -3.019 | 3.019 | -0.748 | -0.127 | 0.127 |
| 65 | 29.75 | 80 | 7.336 | -3.72 | 3.72 | -0.888 | -0.09 | 0.09 |
| 66 | 29.75 | 80.25 | 1.138 | -2.147 | 2.147 | -0.816 | -0.155 | 0.155 |
| 67 | 29.5 | 78.75 | -13.014 | -0.888 | 0.888 | -1.071 | -0.343 | 0.343 |
| 68 | 29.5 | 79 | -9.497 | -1.24 | 1.24 | -1.151 | -0.429 | 0.429 |
| 69 | 29.5 | 79.25 | -10.417 | -0.769 | 0.769 | -1.296 | -0.435 | 0.435 |
| 70 | 29.5 | 79.5 | -0.949 | -0.66 | 0.66 | -0.577 | -0.341 | 0.341 |
| 71 | 29.5 | 79.75 | 1.09 | -0.752 | 0.752 | -1.297 | -0.338 | 0.338 |
| 72 | 29.5 | 80 | 7.324 | -2.197 | 2.197 | -1.113 | -0.119 | 0.119 |
| 73 | 29.5 | 80.25 | 4.874 | -2.392 | 2.392 | -0.681 | -0.116 | 0.116 |
| 74 | 29.25 | 79 | -9.249 | -0.929 | 0.929 | -1.301 | -0.559 | 0.559 |
| 75 | 29.25 | 79.25 | 6.301 | -1.302 | 1.302 | -0.868 | -0.398 | 0.398 |
| 76 | 29.25 | 79.5 | 4.7 | -1.203 | 1.203 | -0.595 | -0.51 | 0.51 |
| 77 | 29.25 | 79.75 | -2.07 | -0.828 | 0.828 | -1.002 | -0.165 | 0.165 |
| 78 | 29.25 | 80 | 1.335 | -1.519 | 1.519 | -0.909 | -0.145 | 0.145 |
| 79 | 29.25 | 80.25 | 1.795 | -1.634 | 1.634 | -0.665 | -0.105 | 0.105 |
| 80 | 29 | 79.25 | 13.036 | -0.814 | 0.814 | -0.102 | -0.676 | 0.676 |
| 81 | 29 | 79.5 | 18.489 | -1.179 | 1.179 | -0.089 | -0.605 | 0.605 |
| 82 | 29 | 79.75 | 3.127 | -0.735 | 0.735 | -0.129 | -0.18 | 0.18 |
| 83 | 29 | 80 | 6.376 | -1.168 | 1.168 | -0.041 | -0.212 | 0.212 |
| 84 | 28.75 | 80 | 3.566 | -0.965 | 0.965 | -0.152 | -0.187 | 0.187 |

| Grid No. | Lat | Long | 1970-2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pre-Monsoon (ITA) |  |  | Monsoon (ITA) |  |  |
|  |  |  | s | CLlower95 | CLupper95 | s | CLlower95 | CLupper95 |
| 1 | 31.25 | 79 | -2.348 | -0.222 | 0.222 | 1.224 | -0.571 | 0.571 |
| 2 | 31.25 | 79.25 | -3.297 | -0.284 | 0.284 | 0.881 | -0.983 | 0.983 |
| 3 | 31 | 78 | -5.263 | -1.476 | 1.476 | -6.577 | -1.645 | 1.645 |
| 4 | 31 | 78.25 | -3.695 | -1.354 | 1.354 | -0.481 | -1.589 | 1.589 |
| 5 | 31 | 78.5 | -1.081 | -0.457 | 0.457 | 3.263 | -0.947 | 0.947 |
| 6 | 31 | 78.75 | -1.81 | -0.658 | 0.658 | 1.067 | -0.895 | 0.895 |
| 7 | 31 | 79 | -2.49 | -0.139 | 0.139 | 2.031 | -0.636 | 0.636 |
| 8 | 31 | 79.25 | -3.155 | -0.197 | 0.197 | 1.475 | -0.666 | 0.666 |
| 9 | 31 | 79.5 | -2.878 | -0.222 | 0.222 | 2.243 | -0.558 | 0.558 |
| 10 | 31 | 79.75 | -2.245 | -0.21 | 0.21 | 5.109 | -0.763 | 0.763 |
| 11 | 30.75 | 77.75 | -1.037 | -0.435 | 0.435 | -6.749 | -0.539 | 0.539 |
| 12 | 30.75 | 78 | -3.21 | -1.908 | 1.908 | -2.522 | -1.197 | 1.197 |
| 13 | 30.75 | 78.25 | -1.336 | -1.524 | 1.524 | 7.503 | -1.97 | 1.97 |
| 14 | 30.75 | 78.5 | 0.121 | -0.476 | 0.476 | 7.654 | -1.373 | 1.373 |
| 15 | 30.75 | 78.75 | -0.749 | -0.256 | 0.256 | 1.357 | -0.768 | 0.768 |
| 16 | 30.75 | 79 | -2.55 | -0.205 | 0.205 | -1.494 | -1 | 1 |
| 17 | 30.75 | 79.25 | -3.128 | -0.251 | 0.251 | 0.985 | -0.641 | 0.641 |
| 18 | 30.75 | 79.5 | -3.206 | -0.365 | 0.365 | 1.516 | -0.546 | 0.546 |
| 19 | 30.75 | 79.75 | -1.684 | -0.183 | 0.183 | 7.902 | -0.462 | 0.462 |
| 20 | 30.75 | 80 | 0.979 | -0.227 | 0.227 | 17.313 | -1.348 | 1.348 |
| 21 | 30.5 | 77.75 | -0.693 | -0.523 | 0.523 | 7.426 | -1.181 | 1.181 |
| 22 | 30.5 | 78 | -1.32 | -0.268 | 0.268 | -1.125 | -1.458 | 1.458 |
| 23 | 30.5 | 78.25 | -0.51 | -0.539 | 0.539 | 4.662 | -1.386 | 1.386 |
| 24 | 30.5 | 78.5 | -0.341 | -0.457 | 0.457 | 4.676 | -0.723 | 0.723 |
| 25 | 30.5 | 78.75 | -0.651 | -0.254 | 0.254 | 2.549 | -0.685 | 0.685 |
| 26 | 30.5 | 79 | -2.512 | -0.406 | 0.406 | -3.979 | -1.062 | 1.062 |
| 27 | 30.5 | 79.25 | -2.042 | -0.209 | 0.209 | 5.544 | -0.817 | 0.817 |
| 28 | 30.5 | 79.5 | -2.644 | -0.194 | 0.194 | 1.716 | -0.65 | 0.65 |
| 29 | 30.5 | 79.75 | -0.112 | -0.208 | 0.208 | 14.288 | -0.491 | 0.491 |
| 30 | 30.5 | 80 | 2.804 | -0.37 | 0.37 | 20.602 | -1.596 | 1.596 |
| 31 | 30.5 | 80.25 | 3.552 | -0.357 | 0.357 | 15.05 | -1.235 | 1.235 |
| 32 | 30.25 | 78 | 0.103 | -0.303 | 0.303 | 1.81 | -0.963 | 0.963 |
| 33 | 30.25 | 78.25 | 0.895 | -0.326 | 0.326 | -2.515 | -0.536 | 0.536 |
| 34 | 30.25 | 78.5 | 0.753 | -0.32 | 0.32 | 6.503 | -0.997 | 0.997 |
| 35 | 30.25 | 78.75 | 0.715 | -0.202 | 0.202 | 8.098 | -0.516 | 0.516 |
| 36 | 30.25 | 79 | -0.605 | -0.266 | 0.266 | 6.562 | -0.751 | 0.751 |
| 37 | 30.25 | 79.25 | -0.765 | -0.228 | 0.228 | 5.966 | -0.546 | 0.546 |
| 38 | 30.25 | 79.5 | -1.579 | -0.269 | 0.269 | 8.666 | -1.081 | 1.081 |
| 39 | 30.25 | 79.75 | 1.727 | -0.347 | 0.347 | 20.256 | -1.236 | 1.236 |
| 40 | 30.25 | 80 | 4.523 | -0.683 | 0.683 | 21.728 | -1.352 | 1.352 |
| 41 | 30.25 | 80.25 | 4.059 | -0.453 | 0.453 | 13.666 | -1.927 | 1.927 |
| 42 | 30.25 | 80.5 | 3.143 | -0.381 | 0.381 | 9.585 | -2.394 | 2.394 |
| 43 | 30.25 | 80.75 | 2.991 | -0.397 | 0.397 | 8.612 | -2.401 | 2.401 |
| 44 | 30.25 | 81 | 2.698 | -0.366 | 0.366 | 7.991 | -2.314 | 2.314 |
| 45 | 30 | 77.75 | 0.677 | -0.161 | 0.161 | 4.823 | -0.577 | 0.577 |
| 46 | 30 | 78 | 0.76 | -0.226 | 0.226 | 3.333 | -0.665 | 0.665 |
| 47 | 30 | 78.25 | -0.001 | -0.187 | 0.187 | -5.55 | 0.764 | -0.764 |
| 48 | 30 | 78.5 | 0.346 | -0.353 | 0.353 | -2.107 | -0.799 | 0.799 |
| 49 | 30 | 78.75 | 0.769 | -0.447 | 0.447 | 1.16 | -0.48 | 0.48 |
| 50 | 30 | 79 | -0.602 | -0.262 | 0.262 | 2.109 | -0.453 | 0.453 |
| 51 | 30 | 79.25 | -0.888 | -0.286 | 0.286 | 3.797 | -0.637 | 0.637 |
| 52 | 30 | 79.5 | 1.48 | -0.22 | 0.22 | 13.913 | -0.656 | 0.656 |
| 53 | 30 | 79.75 | 2.831 | -0.336 | 0.336 | 18.317 | -0.97 | 0.97 |
| 54 | 30 | 80 | 4.907 | -0.75 | 0.75 | 42.123 | -38.744 | 38.744 |
| 55 | 30 | 80.25 | 2.728 | -0.44 | 0.44 | 7.224 | -2.369 | 2.369 |
| 56 | 30 | 80.5 | 2.284 | -0.339 | 0.339 | 5.916 | -2.213 | 2.213 |
| 57 | 29.75 | 78 | -0.026 | -0.169 | 0.169 | -1.914 | -0.568 | 0.568 |
| 58 | 29.75 | 78.25 | -0.62 | -0.187 | 0.187 | 0.346 | -0.828 | 0.828 |
| 59 | 29.75 | 78.5 | -0.577 | -0.52 | 0.52 | -5.284 | -1.106 | 1.106 |
| 60 | 29.75 | 78.75 | -1.025 | -0.288 | 0.288 | -10.043 | -1.034 | 1.034 |
| 61 | 29.75 | 79 | -0.968 | -0.218 | 0.218 | -6.132 | -0.834 | 0.834 |
| 62 | 29.75 | 79.25 | -0.835 | -0.173 | 0.173 | -2.84 | -0.468 | 0.468 |
| 63 | 29.75 | 79.5 | 1.372 | -0.25 | 0.25 | 6.523 | -0.865 | 0.865 |
| 64 | 29.75 | 79.75 | 1.316 | -0.333 | 0.333 | 7.839 | -1.806 | 1.806 |
| 65 | 29.75 | 80 | 1.686 | -0.66 | 0.66 | 5.671 | -2.967 | 2.967 |
| 66 | 29.75 | 80.25 | 1.332 | -0.214 | 0.214 | -0.791 | -1.71 | 1.71 |
| 67 | 29.5 | 78.75 | -1.592 | -0.115 | 0.115 | -9.987 | -0.929 | 0.929 |
| 68 | 29.5 | 79 | -1.241 | -0.199 | 0.199 | -6.798 | -1.268 | 1.268 |
| 69 | 29.5 | 79.25 | -1.26 | -0.196 | 0.196 | -7.421 | -0.67 | 0.67 |
| 70 | 29.5 | 79.5 | -0.31 | -0.125 | 0.125 | -0.403 | -0.594 | 0.594 |
| 71 | 29.5 | 79.75 | -0.063 | -0.173 | 0.173 | 2.039 | -0.507 | 0.507 |
| 72 | 29.5 | 80 | 1.315 | -0.458 | 0.458 | 6.31 | -1.566 | 1.566 |
| 73 | 29.5 | 80.25 | 1.343 | -0.398 | 0.398 | 3.082 | -1.614 | 1.614 |
| 74 | 29.25 | 79 | -1.13 | -0.069 | 0.069 | -6.41 | -0.632 | 0.632 |
| 75 | 29.25 | 79.25 | -0.061 | -0.114 | 0.114 | 6.999 | -1.347 | 1.347 |
| 76 | 29.25 | 79.5 | 0.138 | -0.136 | 0.136 | 4.641 | -1.455 | 1.455 |
| 77 | 29.25 | 79.75 | -0.046 | -0.179 | 0.179 | -1.639 | -0.945 | 0.945 |
| 78 | 29.25 | 80 | 0.325 | -0.126 | 0.126 | 1.387 | -0.977 | 0.977 |
| 79 | 29.25 | 80.25 | 0.31 | -0.169 | 0.169 | 1.547 | -1.389 | 1.389 |
| 80 | 29 | 79.25 | 0.281 | -0.212 | 0.212 | 12.245 | -0.99 | 0.99 |
| 81 | 29 | 79.5 | 0.371 | -0.206 | 0.206 | 17.464 | -1.373 | 1.373 |
| 82 | 29 | 79.75 | 0.329 | -0.458 | 0.458 | 2.193 | -1.372 | 1.372 |
| 83 | 29 | 80 | 0.424 | -0.111 | 0.111 | 5.621 | -1.337 | 1.337 |
| 84 | 28.75 | 80 | -0.274 | -0.236 | 0.236 | 3.922 | -0.961 | 0.961 |

| Grid No. |  | Lat | Long | 1970-2020 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Winter (ITA) |  |  |
|  |  |  |  | s | CLlower95 | CLupper95 |
| 1 |  | 31.25 | 79 | -2.066 | -0.315 | 0.315 |
| 2 |  | 31.25 | 79.25 | -2.138 | -0.263 | 0.263 |
| 3 |  | 31 | 78 | -1.208 | -0.354 | 0.354 |
| 4 |  | 31 | 78.25 | -0.493 | -0.2 | 0.2 |
| 5 |  | 31 | 78.5 | -0.636 | -0.245 | 0.245 |
| 6 |  | 31 | 78.75 | -0.527 | -0.322 | 0.322 |
| 7 |  | 31 | 79 | -1.767 | -0.212 | 0.212 |
| 8 |  | 31 | 79.25 | -1.948 | -0.29 | 0.29 |
| 9 |  | 31 | 79.5 | -1.732 | -0.293 | 0.293 |
| 10 |  | 31 | 79.75 | -1.765 | -0.271 | 0.271 |
| 11 |  | 30.75 | 77.75 | 0.103 | -0.19 | 0.19 |
| 12 |  | 30.75 | 78 | 0.558 | -0.207 | 0.207 |
| 13 |  | 30.75 | 78.25 | 0.757 | -0.228 | 0.228 |
| 14 |  | 30.75 | 78.5 | -0.451 | -0.801 | 0.801 |
| 15 |  | 30.75 | 78.75 | -0.42 | -0.339 | 0.339 |
| 16 |  | 30.75 | 79 | -1.395 | -0.281 | 0.281 |
| 17 |  | 30.75 | 79.25 | -2.006 | -0.26 | 0.26 |
| 18 |  | 30.75 | 79.5 | -2.296 | -0.293 | 0.293 |
| 19 |  | 30.75 | 79.75 | -1.843 | -0.271 | 0.271 |
| 20 |  | 30.75 | 80 | -2.009 | -0.175 | 0.175 |
| 21 |  | 30.5 | 77.75 | 0.697 | -0.15 | 0.15 |
| 22 |  | 30.5 | 78 | -0.618 | -0.135 | 0.135 |
| 23 |  | 30.5 | 78.25 | -0.126 | -0.108 | 0.108 |
| 24 |  | 30.5 | 78.5 | -0.101 | -0.165 | 0.165 |
| 25 |  | 30.5 | 78.75 | -0.181 | -0.214 | 0.214 |
| 26 |  | 30.5 | 79 | -0.933 | -0.181 | 0.181 |
| 27 |  | 30.5 | 79.25 | -0.867 | 0.202 | -0.202 |
| 28 |  | 30.5 | 79.5 | -1.523 | -0.261 | 0.261 |
| 29 |  | 30.5 | 79.75 | -1.515 | -0.4 | 0.4 |
| 30 |  | 30.5 | 80 | -1.359 | -0.274 | 0.274 |
| 31 |  | 30.5 | 80.25 | -0.807 | -0.398 | 0.398 |
| 32 |  | 30.25 | 78 | 0.139 | -0.167 | 0.167 |
| 33 |  | 30.25 | 78.25 | 0.457 | -0.203 | 0.203 |
| 34 |  | 30.25 | 78.5 | 1.05 | -0.114 | 0.114 |
| 35 |  | 30.25 | 78.75 | 0.791 | -0.14 | 0.14 |
| 36 |  | 30.25 | 79 | 0.28 | -0.236 | 0.236 |
| 37 |  | 30.25 | 79.25 | 0.009 | -0.189 | 0.189 |
| 38 |  | 30.25 | 79.5 | -1.557 | -0.287 | 0.287 |
| 39 |  | 30.25 | 79.75 | -2.345 | -0.515 | 0.515 |
| 40 |  | 30.25 | 80 | -1.489 | -0.356 | 0.356 |
| 41 |  | 30.25 | 80.25 | -0.339 | -0.261 | 0.261 |
| 42 |  | 30.25 | 80.5 | 0.389 | -0.396 | 0.396 |
| 43 |  | 30.25 | 80.75 | 0.979 | -0.312 | 0.312 |
| 44 | 30.25 |  | 81 | 1.41 | -0.238 | 0.238 |
| 45 | 30 |  | 77.75 | 0.418 | -0.076 | 0.076 |
| 46 | 30 |  | 78 | 0.677 | -0.181 | 0.181 |
| 47 | 30 |  | 78.25 | 0.484 | -0.201 | 0.201 |
| 48 | 30 |  | 78.5 | 1.482 | -0.195 | 0.195 |
| 49 | 30 |  | 78.75 | 1.466 | -0.153 | 0.153 |
| 50 | 30 |  | 79 | 0.295 | -0.229 | 0.229 |
| 51 | 30 |  | 79.25 | -0.1 | -0.232 | 0.232 |
| 52 | 30 |  | 79.5 | -0.68 | -0.64 | 0.64 |
| 53 | 30 |  | 79.75 | -1.402 | -0.855 | 0.855 |
| 54 | 30 |  | 80 | -0.798 | -0.518 | 0.518 |
| 55 | 30 |  | 80.25 | 1.268 | -0.18 | 0.18 |
| 56 | 30 |  | 80.5 | 1.259 | -0.238 | 0.238 |
| 57 | 29.75 |  | 78 | 0.265 | 0.19 | -0.19 |
| 58 | 29.75 |  | 78.25 | 0.198 | -0.143 | 0.143 |
| 59 | 29.75 |  | 78.5 | 0.679 | -0.213 | 0.213 |
| 60 | 29.75 |  | 78.75 | 0.424 | -0.15 | 0.15 |
| 61 | 29.75 |  | 79 | -0.217 | -0.134 | 0.134 |
| 62 | 29.75 |  | 79.25 | -0.209 | -0.196 | 0.196 |
| 63 | 29.75 |  | 79.5 | 0.792 | -0.169 | 0.169 |
| 64 | 29.75 |  | 79.75 | 0.78 | -0.15 | 0.15 |
| 65 | 29.75 |  | 80 | 0.867 | -0.218 | 0.218 |
| 66 | 29.75 |  | 80.25 | 1.412 | -0.301 | 0.301 |
| 67 | 29.5 |  | 78.75 | -0.364 | -0.09 | 0.09 |
| 68 | 29.5 |  | 79 | -0.307 | -0.105 | 0.105 |
| 69 | 29.5 |  | 79.25 | -0.439 | -0.156 | 0.156 |
| 70 | 29.5 |  | 79.5 | 0.341 | -0.182 | 0.182 |
| 71 | 29.5 |  | 79.75 | 0.411 | -0.134 | 0.134 |
| 72 | 29.5 |  | 80 | 0.813 | -0.151 | 0.151 |
| 73 | 29.5 |  | 80.25 | 1.13 | -0.188 | 0.188 |
| 74 | 29.25 |  | 79 | -0.408 | -0.139 | 0.139 |
| 75 | 29.25 |  | 79.25 | 0.231 | -0.204 | 0.204 |
| 76 | 29.25 |  | 79.5 | 0.515 | -0.163 | 0.163 |
| 77 | 29.25 |  | 79.75 | 0.618 | -0.187 | 0.187 |
| 78 | 29.25 |  | 80 | 0.532 | -0.142 | 0.142 |
| 79 | 29.25 |  | 80.25 | 0.603 | -0.093 | 0.093 |
| 80 | 29 |  | 79.25 | 0.612 | -0.114 | 0.114 |
| 81 | 29 |  | 79.5 | 0.743 | -0.125 | 0.125 |
| 82 | 29 |  | 79.75 | 0.734 | -0.123 | 0.123 |
| 83 | 29 |  | 80 | 0.371 | -0.074 | 0.074 |
| 84 | 28.75 |  | 80 | 0.069 | -0.117 | 0.117 |

| CLlower95 | Lower confidence limit (CL) of the trend slope at 95% significance level |
| --- | --- |
| CLupper95 | Upper confidence limit (CL) of the trend slope at 95% significance level |
| s | Trend slope |

Dataset 5. Percentage of change of annual and seasonal rainfall using ITA from 1970 to 2020.

| Grid No. | Lat | Long | 1970-2020 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ITA (Change in percentage) |  |  |  |  |
|  |  |  | Annual | Pre-Monsoon | Monsoon | Post-monsoon | Winter |
| 1 | 31.25 | 79 | -17.999 | -57.265 | 7.552 | -96.863 | -76.644 |
| 2 | 31.25 | 79.25 | -24.532 | -78.149 | 5.532 | -108.723 | -78.825 |
| 3 | 31 | 78 | -66.634 | -138.91 | -45.632 | -136.101 | -46.182 |
| 4 | 31 | 78.25 | -23.772 | -92.062 | -2.71 | -114.729 | -19.573 |
| 5 | 31 | 78.5 | 2.36 | -27.24 | 18.682 | -83.092 | -25.383 |
| 6 | 31 | 78.75 | -8.053 | -44.305 | 5.805 | -72.694 | -21.822 |
| 7 | 31 | 79 | -12.84 | -60.367 | 11.715 | -87.456 | -66.951 |
| 8 | 31 | 79.25 | -20.382 | -73.815 | 8.722 | -122.362 | -71.517 |
| 9 | 31 | 79.5 | -16.17 | -66.208 | 13.585 | -130.723 | -62.683 |
| 10 | 31 | 79.75 | -1.875 | -51.037 | 30.889 | -122.945 | -63.842 |
| 11 | 30.75 | 77.75 | -36.752 | -30.935 | -36.592 | -143.384 | 3.972 |
| 12 | 30.75 | 78 | -25.026 | -80.596 | -13.906 | -104.814 | 22.123 |
| 13 | 30.75 | 78.25 | 20.221 | -31.915 | 33.451 | -60.278 | 31.066 |
| 14 | 30.75 | 78.5 | 24.398 | 3.268 | 41.505 | -91.794 | -17.2 |
| 15 | 30.75 | 78.75 | -0.783 | -19.147 | 7.233 | -38.086 | -17.4 |
| 16 | 30.75 | 79 | -24.186 | -62.482 | -8.019 | -83.077 | -56.152 |
| 17 | 30.75 | 79.25 | -23.338 | -73.162 | 6.086 | -126.657 | -72.386 |
| 18 | 30.75 | 79.5 | -24.169 | -71.993 | 9.473 | -148.018 | -79.198 |
| 19 | 30.75 | 79.75 | 11.225 | -38.306 | 45.651 | -116.677 | -67.227 |
| 20 | 30.75 | 80 | 48.725 | 20.109 | 78.621 | -76.293 | -68.35 |
| 21 | 30.5 | 77.75 | 22.169 | -23.653 | 29.797 | -47.379 | 32.41 |
| 22 | 30.5 | 78 | -12.916 | -44.455 | -4.603 | -89.719 | -27.408 |
| 23 | 30.5 | 78.25 | 11.293 | -14.12 | 24.796 | -95.669 | -5.038 |
| 24 | 30.5 | 78.5 | 12.715 | -10.54 | 27.275 | -111.958 | -2.113 |
| 25 | 30.5 | 78.75 | 5.685 | -18.191 | 13.344 | -27.097 | -7.966 |
| 26 | 30.5 | 79 | -28.655 | -62.832 | -18.739 | -73.847 | -39.894 |
| 27 | 30.5 | 79.25 | 6.147 | -50.222 | 31.565 | -105.214 | -36.046 |
| 28 | 30.5 | 79.5 | -17.046 | -62.873 | 11.485 | -122.431 | -57.272 |
| 29 | 30.5 | 79.75 | 42.784 | -2.528 | 75.951 | -79.797 | -55.647 |
| 30 | 30.5 | 80 | 62.729 | 57.421 | 83.445 | -56.624 | -47.632 |
| 31 | 30.5 | 80.25 | 43.235 | 71.868 | 49.945 | -53.826 | -28.662 |
| 32 | 30.25 | 78 | 5.022 | 5.37 | 6.647 | -59.677 | 8.342 |
| 33 | 30.25 | 78.25 | -6.912 | 33.1 | -12.533 | -72.672 | 22.173 |
| 34 | 30.25 | 78.5 | 40.636 | 31.073 | 44.355 | -43.234 | 51.606 |
| 35 | 30.25 | 78.75 | 46.896 | 27.268 | 52.152 | 22.383 | 41.204 |
| 36 | 30.25 | 79 | 27.413 | -20.42 | 42.027 | -49.782 | 14.711 |
| 37 | 30.25 | 79.25 | 19.33 | -23.073 | 34.885 | -80.049 | 0.48 |
| 38 | 30.25 | 79.5 | 16.623 | -38.912 | 48.959 | -116.756 | -64.113 |
| 39 | 30.25 | 79.75 | 57.277 | 36.652 | 84.921 | -61.886 | -80.82 |
| 40 | 30.25 | 80 | 60.336 | 87.771 | 70.937 | -34.12 | -50.547 |
| 41 | 30.25 | 80.25 | 40.829 | 81.702 | 43.104 | -47.35 | -12.518 |
| 42 | 30.25 | 80.5 | 29.952 | 65.658 | 29.943 | -62.356 | 15.082 |
| 43 | 30.25 | 80.75 | 28.334 | 62.849 | 26.388 | -60.813 | 39.262 |
| 44 | 30.25 | 81 | 26.988 | 57.27 | 24.043 | -52.585 | 57.322 |
| 45 | 30 | 77.75 | 22.685 | 39.969 | 23.872 | -70.928 | 28.747 |
| 46 | 30 | 78 | 17.984 | 45.041 | 14.736 | 0.375 | 45.754 |
| 47 | 30 | 78.25 | -21.124 | -0.074 | -25.582 | -56.673 | 29.591 |
| 48 | 30 | 78.5 | -4.402 | 16.559 | -11.796 | -97.553 | 84.903 |
| 49 | 30 | 78.75 | 14.722 | 35.293 | 7.417 | -59.302 | 86.998 |
| 50 | 30 | 79 | 5.999 | -24.912 | 13.483 | -71.309 | 17.543 |
| 51 | 30 | 79.25 | 9.155 | -33.601 | 23.821 | -110.295 | -5.963 |
| 52 | 30 | 79.5 | 59.09 | 49.33 | 77.493 | -77.538 | -33.685 |
| 53 | 30 | 79.75 | 56.85 | 68.846 | 70.458 | -50.995 | -56.325 |
| 54 | 30 | 80 | 59.373 | 95.09 | 44.235 | -21.929 | -28.787 |
| 55 | 30 | 80.25 | 24.911 | 58.736 | 22.16 | -66.574 | 52.449 |
| 56 | 30 | 80.5 | 20.745 | 50.25 | 18.387 | -73.833 | 52.938 |
| 57 | 29.75 | 78 | -9.58 | -1.717 | -10.14 | -71.944 | 20.639 |
| 58 | 29.75 | 78.25 | -6.947 | -47.096 | 1.955 | -112.64 | 16.959 |
| 59 | 29.75 | 78.5 | -26.56 | -35.938 | -25.736 | -168.047 | 51.668 |
| 60 | 29.75 | 78.75 | -55.267 | -68.153 | -55.93 | -163.942 | 33.553 |
| 61 | 29.75 | 79 | -39.528 | -51.621 | -34.804 | -148.999 | -14.963 |
| 62 | 29.75 | 79.25 | -23.232 | -35.599 | -16.764 | -133.1 | -12.663 |
| 63 | 29.75 | 79.5 | 39.442 | 55.425 | 40.838 | -52.09 | 48.661 |
| 64 | 29.75 | 79.75 | 35.539 | 41.876 | 39.01 | -86.335 | 44.616 |
| 65 | 29.75 | 80 | 22.238 | 41.834 | 21.77 | -86.849 | 45.996 |
| 66 | 29.75 | 80.25 | 2.908 | 31.054 | -2.526 | -62.145 | 62.778 |
| 67 | 29.5 | 78.75 | -66.243 | -146.368 | -59.43 | -149.641 | -35.108 |
| 68 | 29.5 | 79 | -44.967 | -87.842 | -38.487 | -136.802 | -25.529 |
| 69 | 29.5 | 79.25 | -36.281 | -44.544 | -32.204 | -130.707 | -23.783 |
| 70 | 29.5 | 79.5 | -3.913 | -10.065 | -2.247 | -49.266 | 16.582 |
| 71 | 29.5 | 79.75 | 4.62 | -2.115 | 11.442 | -134.076 | 22.26 |
| 72 | 29.5 | 80 | 26.682 | 37.518 | 29.832 | -109.582 | 45.667 |
| 73 | 29.5 | 80.25 | 14.731 | 35.412 | 11.748 | -61.198 | 58.18 |
| 74 | 29.25 | 79 | -44.229 | -88.153 | -36.05 | -173.302 | -37.129 |
| 75 | 29.25 | 79.25 | 20.46 | -2.952 | 26.643 | -91.564 | 15.136 |
| 76 | 29.25 | 79.5 | 16.274 | 5.474 | 19.734 | -55.313 | 29.286 |
| 77 | 29.25 | 79.75 | -8.442 | -1.876 | -8.514 | -95.205 | 35.564 |
| 78 | 29.25 | 80 | 5.087 | 12.818 | 6.613 | -83.641 | 32.168 |
| 79 | 29.25 | 80.25 | 6.213 | 11.558 | 6.569 | -62.567 | 37.891 |
| 80 | 29 | 79.25 | 51.613 | 18.723 | 56.645 | -10.658 | 51.607 |
| 81 | 29 | 79.5 | 78.118 | 28.29 | 85.595 | -10.801 | 65.577 |
| 82 | 29 | 79.75 | 11.087 | 20.955 | 8.957 | -12.826 | 64.254 |
| 83 | 29 | 80 | 21.738 | 27.151 | 22.08 | -3.763 | 30.794 |
| 84 | 28.75 | 80 | 10.712 | -17.828 | 13.363 | -12.069 | 6.045 |

Dataset 6. Percentage of change of annual and seasonal rainfall using Mann-Kendall between 1970 and 2020.

| Grid No. | Lat | Long | 1970-2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mann-Kendall (Change in percentage) |  |  |  |  |  |
|  |  |  | Annual | Pre-Monsoon | Monsoon | Post-monsoon |  | Winter |
| 1 | 31.25 | 79 | -10.46 | -36.922 | 4.702 | -58.361 |  | -65.808 |
| 2 | 31.25 | 79.25 | -16.457 | -45.978 | 2.71 | -54.469 |  | -75.953 |
| 3 | 31 | 78 | -52.059 | -39.018 | -55.282 | -59.844 |  | -59.76 |
| 4 | 31 | 78.25 | -14.069 | -19.311 | -14.203 | -30.856 |  | -21.856 |
| 5 | 31 | 78.5 | 2.806 | 1.479 | 1.506 | -41.82 |  | -17.833 |
| 6 | 31 | 78.75 | -0.234 | 0.175 | -4.027 | -31.477 |  | -5.18 |
| 7 | 31 | 79 | -4.017 | -34.105 | 7.251 | -39.621 |  | -56.14 |
| 8 | 31 | 79.25 | -7.114 | -46.788 | 13.351 | -66.602 |  | -69.763 |
| 9 | 31 | 79.5 | -2.722 | -40.203 | 15.858 | -66.19 |  | -68.381 |
| 10 | 31 | 79.75 | 2.803 | -33.291 | 28.44 | -66.036 |  | -70.171 |
| 11 | 30.75 | 77.75 | -30.112 | 2.506 | -33.598 | -55.132 |  | -2.46 |
| 12 | 30.75 | 78 | -17.137 | -9.432 | -15.748 | -46.262 |  | 13.352 |
| 13 | 30.75 | 78.25 | 17.497 | 14.331 | 18.797 | -5.029 |  | 25.636 |
| 14 | 30.75 | 78.5 | -5.225 | 13.247 | -5.067 | -46.476 |  | 11.442 |
| 15 | 30.75 | 78.75 | 5.479 | 3.877 | 2.901 | -12.312 |  | 8.795 |
| 16 | 30.75 | 79 | -9.453 | -26.543 | 5.597 | -35.949 |  | -39.255 |
| 17 | 30.75 | 79.25 | -7.491 | -43.402 | 13.104 | -67.53 |  | -71.727 |
| 18 | 30.75 | 79.5 | -11.746 | -44.308 | 10.522 | -71.109 |  | -84.002 |
| 19 | 30.75 | 79.75 | 13.646 | -24.919 | 44.941 | -70.165 |  | -75.847 |
| 20 | 30.75 | 80 | 29.625 | 3.081 | 53.76 | -55.916 |  | -64.139 |
| 21 | 30.5 | 77.75 | 21.312 | 32.693 | 25.358 | -22.327 |  | 29.15 |
| 22 | 30.5 | 78 | -5.075 | -11.981 | -6.013 | -36.638 |  | 1.9 |
| 23 | 30.5 | 78.25 | -1.761 | -3.336 | -3.438 | -51.486 |  | 7.412 |
| 24 | 30.5 | 78.5 | -0.591 | -2.883 | 3.889 | -62.739 |  | 15.557 |
| 25 | 30.5 | 78.75 | 22.886 | 20.943 | 24.461 | 2.646 |  | 14.644 |
| 26 | 30.5 | 79 | 2.846 | -13.717 | 11.274 | -6.847 |  | -24.728 |
| 27 | 30.5 | 79.25 | 12.796 | -31.148 | 32.771 | -55.31 |  | -45.759 |
| 28 | 30.5 | 79.5 | -6.61 | -37.001 | 10.92 | -70.765 |  | -72.754 |
| 29 | 30.5 | 79.75 | 40.873 | 5.552 | 70.715 | -66.052 |  | -50.607 |
| 30 | 30.5 | 80 | 35.943 | 21.159 | 56.219 | -58.297 |  | -38.743 |
| 31 | 30.5 | 80.25 | 16.519 | 25.143 | 28.811 | -58.473 |  | -24.874 |
| 32 | 30.25 | 78 | 16.586 | 57.229 | 11.02 | -2.138 |  | 29.044 |
| 33 | 30.25 | 78.25 | 13.377 | 64.881 | 0.958 | -13.638 |  | 28.406 |
| 34 | 30.25 | 78.5 | 38.013 | 36.932 | 39.82 | -40.624 |  | 60.961 |
| 35 | 30.25 | 78.75 | 40.386 | 47.415 | 47.246 | -17.853 |  | 39.547 |
| 36 | 30.25 | 79 | 43.436 | 43.501 | 60.234 | 19.429 |  | 4.079 |
| 37 | 30.25 | 79.25 | 38.433 | 25.015 | 50.05 | -8.239 |  | -9.876 |
| 38 | 30.25 | 79.5 | 24.153 | -22.515 | 58.119 | -81.175 |  | -68.919 |
| 39 | 30.25 | 79.75 | 18.631 | 5.305 | 55.433 | -56.434 |  | -52.047 |
| 40 | 30.25 | 80 | 16.943 | 27.706 | 29.825 | -51.951 |  | -39.258 |
| 41 | 30.25 | 80.25 | 6.251 | 26.304 | 11.604 | -55.889 |  | -20.193 |
| 42 | 30.25 | 80.5 | 8.275 | 25.068 | 11.741 | -63.537 |  | -11.915 |
| 43 | 30.25 | 80.75 | 11.542 | 24.038 | 12.141 | -62.686 |  | -2.247 |
| 44 | 30.25 | 81 | 14.469 | 24.763 | 14.742 | -60.683 | 6.196 |  |
| 45 | 30 | 77.75 | 19.451 | 48.954 | 14.353 | -19.964 | 10.319 |  |
| 46 | 30 | 78 | 21.95 | 75.22 | 18.354 | 7.489 | 24.341 |  |
| 47 | 30 | 78.25 | 1.02 | 43.398 | -3.854 | -24.042 | 18.351 |  |
| 48 | 30 | 78.5 | -4.493 | 24.405 | -15.9 | -48.583 | 45.385 |  |
| 49 | 30 | 78.75 | 13.665 | 39.022 | 4.099 | -23.555 | 53.642 |  |
| 50 | 30 | 79 | 7.79 | 25.364 | 14.169 | -47.75 | 1.921 |  |
| 51 | 30 | 79.25 | 14.329 | 15.436 | 24.702 | -48.745 | -14.375 |  |
| 52 | 30 | 79.5 | 51.065 | 31.02 | 75.694 | -64.618 | 10.568 |  |
| 53 | 30 | 79.75 | 28.621 | 18.726 | 44.604 | -60.026 | -7.143 |  |
| 54 | 30 | 80 | 10.194 | 32.246 | 16.487 | -41.485 | -8.636 |  |
| 55 | 30 | 80.25 | 9.366 | 26.713 | 11.018 | -71.777 | 4.612 |  |
| 56 | 30 | 80.5 | 10.858 | 29.308 | 10.201 | -70.966 | 7.978 |  |
| 57 | 29.75 | 78 | 3.243 | 57.635 | 1.589 | -37.79 | 13.116 |  |
| 58 | 29.75 | 78.25 | 18.775 | 24.057 | 23.309 | -34.4 | 32.38 |  |
| 59 | 29.75 | 78.5 | -32.067 | 12.458 | -34.566 | -51.909 | 10.652 |  |
| 60 | 29.75 | 78.75 | -46.884 | -14.033 | -48.596 | -74.403 | 3.334 |  |
| 61 | 29.75 | 79 | -36.433 | -1.777 | -47.674 | -90.344 | -12.385 |  |
| 62 | 29.75 | 79.25 | -24.345 | -3.999 | -26.907 | -83.422 | -15.342 |  |
| 63 | 29.75 | 79.5 | 28.765 | 70.726 | 29.831 | -30.08 | 65.306 |  |
| 64 | 29.75 | 79.75 | 14.133 | 37.979 | 10.517 | -43.815 | 43.794 |  |
| 65 | 29.75 | 80 | 8.695 | 28.254 | -2.961 | -16.414 | 32.528 |  |
| 66 | 29.75 | 80.25 | 2.121 | 17.675 | -2.643 | -66.405 | 22.557 |  |
| 67 | 29.5 | 78.75 | -56.691 | -40.175 | -55.414 | -75.727 | -30.9 |  |
| 68 | 29.5 | 79 | -46.577 | -20.728 | -52.624 | -66.555 | -14.589 |  |
| 69 | 29.5 | 79.25 | -28.639 | -14.142 | -36.004 | -78.05 | -22.57 |  |
| 70 | 29.5 | 79.5 | -3.012 | 2.705 | -5.575 | -68.295 | 15.377 |  |
| 71 | 29.5 | 79.75 | 9.375 | 23.843 | 6.103 | -49.61 | 34.594 |  |
| 72 | 29.5 | 80 | 19.935 | 64.299 | 13.958 | -29.534 | 70.633 |  |
| 73 | 29.5 | 80.25 | 13.199 | 34.984 | 1.276 | -30.186 | 51.852 |  |
| 74 | 29.25 | 79 | -38.063 | -12.711 | -42.964 | -49.54 | -8.145 |  |
| 75 | 29.25 | 79.25 | 28.034 | 41.427 | 24.114 | -44.091 | 28.572 |  |
| 76 | 29.25 | 79.5 | 26.42 | 32.73 | 22.627 | -37.216 | 37.089 |  |
| 77 | 29.25 | 79.75 | -9.599 | 17.675 | -15.419 | -37.999 | 30.099 |  |
| 78 | 29.25 | 80 | 1.801 | 27.087 | 0.778 | -25.559 | 34.162 |  |
| 79 | 29.25 | 80.25 | 5.73 | 32.874 | 1.583 | -36.688 | 45.856 |  |
| 80 | 29 | 79.25 | 51.025 | 48.847 | 49.151 | -10.968 | 40.173 |  |
| 81 | 29 | 79.5 | 105.696 | 97.733 | 106.051 | 14.683 | 82.37 |  |
| 82 | 29 | 79.75 | 12.714 | 61.313 | 6.562 | -9.959 | 63.497 |  |
| 83 | 29 | 80 | 27.879 | 52.124 | 29.669 | -7.54 | 34.557 |  |
| 84 | 28.75 | 80 | 14.62 | 19.268 | 16.584 | -27.545 | 27.837 |  |

Dataset 7. Percentage of change of annual and seasonal rainfall using Modified Mann-Kendall from 1970 to 2020.

| Grid No. | Lat | Long | 1970-2020 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Modified Mann-Kendall (Change in percentage) |  |  |  |  |
|  |  |  | Annual | Pre-Monsoon | Monsoon | Post-monsoon | Winter |
| 1 | 31.25 | 79 | -13.675 | -31.49 | 5.656 | -61.663 | -68.511 |
| 2 | 31.25 | 79.25 | -9.675 | -34.203 | 3.451 | -57.019 | -60.232 |
| 3 | 31 | 78 | -48.269 | -40.91 | -55.608 | -70.785 | -49.679 |
| 4 | 31 | 78.25 | -15.401 | -26.003 | -13.219 | -30.911 | -22.282 |
| 5 | 31 | 78.5 | 3.636 | -0.918 | 7.732 | -44.151 | -19.007 |
| 6 | 31 | 78.75 | -2.324 | -9.197 | -1.237 | -31.21 | -2.489 |
| 7 | 31 | 79 | -6.687 | -30.042 | 6.659 | -49.491 | -48.569 |
| 8 | 31 | 79.25 | -6.478 | -37.852 | 9.597 | -64.587 | -50.004 |
| 9 | 31 | 79.5 | -6.455 | -36.269 | 10.906 | -63.388 | -53.678 |
| 10 | 31 | 79.75 | -2.056 | -29.102 | 21.929 | -62.632 | -52.942 |
| 11 | 30.75 | 77.75 | -35.377 | 5.412 | -28.07 | -33.216 | 1.541 |
| 12 | 30.75 | 78 | -22.912 | -17.952 | -16.104 | -35.777 | 11.956 |
| 13 | 30.75 | 78.25 | 20.63 | 14.428 | 24.296 | -11.014 | 28.515 |
| 14 | 30.75 | 78.5 | -1.959 | 13.109 | -1.231 | -34.639 | 9.68 |
| 15 | 30.75 | 78.75 | 2.839 | -1.059 | 3.21 | -24.931 | 11.136 |
| 16 | 30.75 | 79 | -4.615 | -29.869 | 3.725 | -43.411 | -32.091 |
| 17 | 30.75 | 79.25 | -4.659 | -38.842 | 7.874 | -64.921 | -52.937 |
| 18 | 30.75 | 79.5 | -10.704 | -39.403 | 6.401 | -57.458 | -50.795 |
| 19 | 30.75 | 79.75 | 7.458 | -25.17 | 38.075 | -65.43 | -56.635 |
| 20 | 30.75 | 80 | 15.063 | -1.8 | 31.618 | -52.126 | -29.061 |
| 21 | 30.5 | 77.75 | 25.652 | 23.938 | 24.855 | 3.038 | 30.98 |
| 22 | 30.5 | 78 | -2.623 | -5.082 | -1.378 | -29.081 | 4.712 |
| 23 | 30.5 | 78.25 | -0.835 | -3.292 | -1.702 | -51.612 | 9.07 |
| 24 | 30.5 | 78.5 | -1.813 | -2.328 | 0.603 | -55.603 | 3.445 |
| 25 | 30.5 | 78.75 | 22.772 | 10.457 | 21.104 | -11.15 | 21.209 |
| 26 | 30.5 | 79 | 2.489 | -26.772 | 8.539 | -14.153 | -22.027 |
| 27 | 30.5 | 79.25 | 9.802 | -40.569 | 25.443 | -67.735 | -40.423 |
| 28 | 30.5 | 79.5 | -7.794 | -32.051 | 9.046 | -75.406 | -57.527 |
| 29 | 30.5 | 79.75 | 29.628 | -7.505 | 53.668 | -57.361 | -36.261 |
| 30 | 30.5 | 80 | 13.374 | 8.555 | 25.446 | -48.991 | -17.988 |
| 31 | 30.5 | 80.25 | 10.53 | 22.05 | 11.713 | -55.932 | -1.096 |
| 32 | 30.25 | 78 | 17.685 | 48.042 | 15.296 | 15.462 | 23.784 |
| 33 | 30.25 | 78.25 | 17.278 | 43.48 | 6.42 | -21.461 | 29.665 |
| 34 | 30.25 | 78.5 | 33.484 | 34.961 | 30.543 | -53.474 | 69.496 |
| 35 | 30.25 | 78.75 | 46.229 | 44.297 | 49.956 | -28.893 | 51.237 |
| 36 | 30.25 | 79 | 44.895 | 38.064 | 57.345 | 14.261 | 20.028 |
| 37 | 30.25 | 79.25 | 40.829 | 22.615 | 45.715 | -14.229 | 2.205 |
| 38 | 30.25 | 79.5 | 17.96 | -23.079 | 45.355 | -76.153 | -44.7 |
| 39 | 30.25 | 79.75 | 15.982 | 17.003 | 28.999 | -50.394 | -10.361 |
| 40 | 30.25 | 80 | 13.75 | 24.292 | 19.403 | -43.293 | -2.192 |
| 41 | 30.25 | 80.25 | 11.111 | 22.525 | 8.84 | -49.756 | -8.507 |
| 42 | 30.25 | 80.5 | 13.83 | 24.249 | 6.248 | -57.85 | 5.489 |
| 43 | 30.25 | 80.75 | 9.803 | 22.823 | 6.885 | -58.546 | 9.643 |
| 44 | 30.25 | 81 | 9.007 | 16.09 | 8.28 | -57.829 | 8.71 |
| 45 | 30 | 77.75 | 17.923 | 53.213 | 13.54 | -21.084 | 15.274 |
| 46 | 30 | 78 | 25.041 | 68.946 | 21.343 | 7.705 | 25.033 |
| 47 | 30 | 78.25 | 7.944 | 32.233 | 0.162 | -28.472 | 34.862 |
| 48 | 30 | 78.5 | -3.91 | 19.694 | -13.868 | -58.495 | 66.14 |
| 49 | 30 | 78.75 | 14.813 | 37.59 | 4.15 | -27.912 | 72.78 |
| 50 | 30 | 79 | 10.868 | 22.266 | 16.896 | -51.78 | 15.381 |
| 51 | 30 | 79.25 | 15.466 | 10.842 | 25.46 | -49.203 | 3.028 |
| 52 | 30 | 79.5 | 46.575 | 26.467 | 64.371 | -59.001 | 11.42 |
| 53 | 30 | 79.75 | 26.33 | 27.593 | 28.54 | -56.757 | 7.366 |
| 54 | 30 | 80 | 13.866 | 30.643 | 8.912 | -32.102 | 9.172 |
| 55 | 30 | 80.25 | 8.134 | 20.475 | 8.18 | -68.392 | 10.815 |
| 56 | 30 | 80.5 | 11.658 | 19.89 | 6.473 | -59.876 | 7.213 |
| 57 | 29.75 | 78 | 4.64 | 55.306 | -1.459 | -39.354 | 14.079 |
| 58 | 29.75 | 78.25 | 20.205 | 29.556 | 21.869 | -35.256 | 32.991 |
| 59 | 29.75 | 78.5 | -25.417 | 15.35 | -26.994 | -55.051 | 23.34 |
| 60 | 29.75 | 78.75 | -38.554 | -9.247 | -44.505 | -75.917 | 6.992 |
| 61 | 29.75 | 79 | -34.974 | 5.481 | -39.803 | -97.146 | 2.827 |
| 62 | 29.75 | 79.25 | -19.113 | 4.402 | -19.596 | -93.622 | 0.342 |
| 63 | 29.75 | 79.5 | 28.663 | 27.866 | 30.943 | -30.17 | 45.031 |
| 64 | 29.75 | 79.75 | 10.037 | 25.453 | 9.651 | -30.554 | 31.017 |
| 65 | 29.75 | 80 | 15.247 | 27.52 | 12.533 | -2.656 | 33.246 |
| 66 | 29.75 | 80.25 | 6.781 | 21.339 | 4.84 | -61.561 | 23.641 |
| 67 | 29.5 | 78.75 | -55.519 | -27.193 | -58.846 | -81.936 | -9.826 |
| 68 | 29.5 | 79 | -34.344 | -3.405 | -41.439 | -82.02 | -2.976 |
| 69 | 29.5 | 79.25 | -21.231 | -8.261 | -26.427 | -86.976 | -7.88 |
| 70 | 29.5 | 79.5 | -5.063 | 5.699 | -8.018 | -64.906 | 8.594 |
| 71 | 29.5 | 79.75 | 5.477 | 15.444 | 0.443 | -48.073 | 24.364 |
| 72 | 29.5 | 80 | -2.088 | 36.789 | -1.547 | -36.966 | 53.77 |
| 73 | 29.5 | 80.25 | 4.468 | 26.434 | 4.83 | -27.37 | 37.841 |
| 74 | 29.25 | 79 | -35.783 | 0.85 | -41.181 | -64.927 | -7.59 |
| 75 | 29.25 | 79.25 | 27.236 | 40.653 | 22.204 | -44.124 | 33.564 |
| 76 | 29.25 | 79.5 | 30.509 | 31.408 | 25.237 | -40.308 | 38.769 |
| 77 | 29.25 | 79.75 | -10.211 | 15.336 | -15.92 | -40.742 | 35.311 |
| 78 | 29.25 | 80 | -7.581 | 19.656 | -8.859 | -28.822 | 39.619 |
| 79 | 29.25 | 80.25 | -3.835 | 24.058 | -0.35 | -32.3 | 35.374 |
| 80 | 29 | 79.25 | 61.017 | 39.183 | 65.445 | -10.247 | 23.796 |
| 81 | 29 | 79.5 | 96.471 | 93.848 | 106.52 | 17.082 | 62.564 |
| 82 | 29 | 79.75 | 7.03 | 50.115 | 2.537 | -11.217 | 71.944 |
| 83 | 29 | 80 | 15.852 | 36.274 | 18.394 | -11.96 | 45.703 |
| 84 | 28.75 | 80 | 10.698 | 4.968 | 13.338 | -32.265 | 27.909 |

# Data availability statement

The authors confirm that the data supporting the findings of this study are available within the article or its supplementary materials.

# Disclosure statement (conflict of interest statement)

There are no relevant financial or non-financial competing interests to report.

References

Anbalagan, R., 1996. Hazards of erosion and sedimentation due to cloudburst in small catchment: a case study from Kumaun Himalaya, India. Ecohydrology of High Mountain Areas, 313.

Das, P. K., 2013. ‘The Himalayan Tsunami’- Cloudburst, Flash Flood & Death Toll: A Geographical Postmortem. IOSR Journal Of Environmental Science, Toxicology And Food Technology, 7(2):33–45. DOI: 10.9790/2402-0723345.

Dimri, A. P. et al., 2017. Cloudbursts in Indian Himalayas: A Review. Earth-Science Reviews, 168: 1–23.

Guhathakurta, P., Sreejith, O. P., and Menon, P. A., 2011. Impact of climate change on extreme rainfall events and flood risk in India. Journal of Earth System Science, 120(3), 359–373. DOI:10.1007/s12040-011-0082-5.

Gupta, V., Dobhal, D. P., and Vaideswaran, S. C., 2013. August 2012 cloudburst and subsequent flash flood in the Asi Ganga, a tributary of the Bhagirathi river, Garhwal Himalaya, India. Current science, 105(2), 249-253.

Joshi, V., and Maikhuri, R. K., 1997. Cloudburst: a natural calamity-a case study from Garhwal Himalaya, UP. Journal of Indian Building Congress, Vol. 4, pp. 207-19.

Joshi, V., and Kumar, K., 2006. Extreme rainfall events and associated natural hazards in Alaknanda valley, Indian Himalayan region. Journal of Mountain Science, 3(3), 228-236. DOI: 10.1007/s11629-006-0228-0.

Khanduri, S., Sajwan, K. S., and Rawat, A., 2018. Disastrous Events on Kelash-Mansarowar Route, Dharchula Tehsil in Pithoragarh District, Uttarakhand in India. Journal of Earth Science & Climatic Change, 9(4), 1-4. DOI: 10.4172/2157-7617.1000463.

Khanduri, S., 2020. Cloudbursts Over Indian Sub-continent of Uttarakhand Himalaya: A Traditional Habitation Input from Bansoli, District-Chamoli, India. International Journal of Earth Sciences Knowledge and Applications, 2(2), 48-63.

Kundu, S., Khare, D., Mondal, A., and Mishra, P. K., 2015. Analysis of spatial and temporal variation in rainfall trend of Madhya Pradesh, India (1901–2011). Environmental Earth Sciences, 73(12), 8197–8216. DOI:10.1007/s12665-014-3978-y.

Manandhar, B., 2010. Flood plain analysis and risk assessment of Lothar Khola. Master of Science Thesis in Watershed Management. Tribhuvan University Institute of Forestry Pokhara, Nepal.

Mondal, A., Khare, D., and Kundu, S., 2015. Spatial and temporal analysis of rainfall and temperature trend of India. Theoretical and applied climatology, 122(1), 143-158. DOI:10.1007/s00704-014-1283-z.

Narayanan, P., Basistha, A., Sarkar, S., and Kamna, S., 2013. Trend analysis and ARIMA modelling of pre-monsoon rainfall data for western India. Comptes Rendus - Geoscience, 345(1), 22–27. DOI:10.1016/j.crte.2012.12.001.

Pandey, V. K., and Mishra, A., 2015. Causes and Disaster Risk Reduction Measures for Hydrometerological Disaster in Uttarakhand, India: An Overview. International Journal of Current Research in Science and Technology, 1(3), 61-80.

Pandey, V. K., and Mishra, A., 2018. Trends of Hydro-Meteorological Disaster in Uttarakhand, India. International Journal of Current Research in Science and Technology, 4(12), 1-7.

Pandey, K., and Vishwakarma, D. K., 2019. 7 Flash Floods Cause and Remedial Measures for Their Control in Hilly Regions. Applied Agricultural Practices for Mitigating Climate Change, 2, 77. DOI: 10.1201/9780429326400-7.

Rana, N., Singh, S., Sundriyal, Y. P., and Juyal, N., 2013. Recent and past floods in the Alaknanda valley: causes and consequences. Current Science, 105(9), 1209-1212.

Sajwan, K. S., Khanduri, S., and Bhaisora, B., 2017. Some significant aspect of cloudburst with especial reference to devastating landslides at Bastari, Naulra and Didihat region, Pithoragarh district, Kumaon Himalaya, Uttarakhand. International Journal Current Research, 9(07), 54255-54262.

Sajwan, K. S., and Khanduri, S., 2018. Investigation of Hydrometrological Disaster Affected Malpa and Mangti Area, Pithoragarh District, Uttarakhand, India. Journal of Geography and Natural Disasters, 8(2), 1-7.

Sati, V.P., 2020. Increasing Incidences of Cloudburst Triggered Debris-Flows/Flash Floods in Garhwal Himalaya, India.

Singh, A. K., Tripathi, J. N., Singh, K. K., Singh, V., and Sateesh, M., 2019. Comparison of different satellite-derived rainfall products with IMD gridded data over Indian meteorological subdivisions during Indian Summer Monsoon (ISM) 2016 at weekly temporal resolution. Journal of Hydrology, 575, 1371-1379. DOI: 10.1016/j.jhydrol.2019.02.016

South Asia Network on Dam, Rivers and People (SANDARP)-Uttrakhand Cloudburst Incidents 2018, accessed 15 September 2020, <https://sandrp.in/2018/07/21/uttrakhand-cloudburst-incidents-2018/>

South Asia Network on Dam, Rivers and People (SANDARP)-Uttarakhand Cloud Bursts in Monsoon 2019: No Doppler Radars Six Years Since 2013 Disaster, accessed 1 October 2020,                             < https://sandrp.in/2019/12/11/uttarakhand-cloud-bursts-in-monsoon-2019-no-doppler-radars-six-years-since-2013-disaster/>